|  | OceanStor Dorado 6.1.6.SPH20 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Patch Notes |  |  |  |
|  | Issue | 01 |  |  |
|  | Date | 2024-08-30 |  |  |
|  | HUAWEI TECHNOLOGIES CO., LTD. |  |  |  |

Copyright © Huawei Technologies Co., Ltd. 2024. All rights reserved. No part of this document may be reproduced or transmitted in any form or by any means without prior written consent of Huawei Technologies Co., Ltd. Trademarks and Permissions and other Huawei trademarks are trademarks of Huawei Technologies Co., Ltd. All other trademarks and trade names mentioned in this document are the property of their respective holders. Notice The purchased products, services and features are stipulated by the contract made between Huawei and the customer. All or part of the products, services and features described in this document may not be within the purchase scope or the usage scope. Unless otherwise specified in the contract, all statements, information, and recommendations in this document are provided "AS IS" without warranties, guarantees or representations of any kind, either express or implied. The information in this document is subject to change without notice. Every effort has been made in the preparation of this document to ensure accuracy of the contents, but all statements, information, and recommendations in this document do not constitute a warranty of any kind, express or implied.

| Huawei Technologies Co., Ltd. |  |
| --- | --- |
| Address: | Huawei Industrial Base Bantian, Longgang Shenzhen 518129 People's Republic of China |
| Website: | https://e.huawei.com |

Security Declaration

Product Lifecycle Huawei's regulations on product lifecycle are subject to the Product End of Life Policy. For details about this policy, visit the following web page: https://support.huawei.com/ecolumnsweb/en/warranty-policy Vulnerability Huawei's regulations on product vulnerability management are subject to the Vul. Response Process. For details about this process, visit the following web page: https://www.huawei.com/en/psirt/vul-response-process For vulnerability information, enterprise customers can visit the following web page: https://securitybulletin.huawei.com/enterprise/en/security-advisory Initial Digital Certificate The Initial digital certificates on Huawei devices are subject to the Rights and Responsibilities of Initial Digital Certificates on Huawei Devices. For details about this document, visit the following web page: https://support.huawei.com/enterprise/en/bulletins-service/ENEWS2000015789 Huawei Enterprise End User License Agreement This agreement is the end user license agreement between you (an individual, company, or any other entity) and Huawei for the use of the Huawei Software. Your use of the Huawei Software will be deemed as your acceptance of the terms mentioned in this agreement. For details about this agreement, visit the following web page: https://e.huawei.com/en/about/eula Lifecycle of Product Documentation Huawei after-sales user documentation is subject to the Product Documentation Lifecycle Policy. For details about this policy, visit the following web page: https://support.huawei.com/enterprise/en/bulletins-website/ENEWS2000017761

Contents

1 Change History	1

2 Patch Overview	2

2.1 Basic Information	2

2.2 Impact of Patch Installation	3

2.3 Version Compatibility Requirements	3

2.4 Virus Scan Result	3

3 Resolved Issues	4

3.1 SPH20 Resolved Issues	4

3.2 SPH18 Resolved Issues	9

3.3 SPH17 Resolved Issues	16

3.4 SPH15 Resolved Issues	19

3.5 SPH13 Resolved Issues	20

3.6 SPH10 Resolved Issues	21

3.7 SPH7 Resolved Issues	29

3.8 SPH6 Resolved Issues	32

3.9 SPH3 Resolved Issues	35

3.10 SPH2 Resolved Issues	36

4 Third-Party Software Vulnerability Fixes	38

# Change History

| Issue | Date | Description |
| --- | --- | --- |
| 01 | 2024-08-30 | This issue is the first official release. |

# Patch Overview

2.1  Basic Information

2.2  Impact of Patch Installation

2.3  Version Compatibility Requirements

2.4  Virus Scan Result

## Basic Information

Basic information about OceanStor Dorado 6.1.6.SPH20

| Patch Name | OceanStor Dorado 6.1.6.SPH20 |
| --- | --- |
| Product Version | OceanStor Dorado 6.1.6 |
| Release Date | 2024-08-30 |
| Compatibility with Other Patches | OceanStor Dorado 6.1.6.SPH18 OceanStor Dorado 6.1.6.SPH17 OceanStor Dorado 6.1.6.SPH15 OceanStor Dorado 6.1.6.SPH13 OceanStor Dorado 6.1.6.SPH10 OceanStor Dorado 6.1.6.SPH7 OceanStor Dorado 6.1.6.SPH6 OceanStor Dorado 6.1.6.SPH3 OceanStor Dorado 6.1.6.SPH2 |

## Impact of Patch Installation

Impact on the System During Installation

Impact on services: Configuration operations on the storage system are prohibited (including REST or CLI configuration commands delivered by OpenStack Cinder, SMI-S, and other plug-in interfaces). Other services or operations are not affected.

Impact on network communication: None

Impact on the System After Installation

None

## Version Compatibility Requirements

The mapping relationships are consistent with those of OceanStor Dorado 6.1.6.

## Virus Scan Result

The software package of OceanStor Dorado 6.1.6.SPH20 storage system has been scanned by Kav, Avira, and OSCE antivirus software.

Table Virus scan result lists the scan result.

| Antivirus Software Name | Result |
| --- | --- |
| Avira | OK |
| Kav | OK |
| OSCE | OK |

# Resolved Issues

3.1  SPH20 Resolved Issues

3.2  SPH18 Resolved Issues

3.3  SPH17 Resolved Issues

3.4  SPH15 Resolved Issues

3.5  SPH13 Resolved Issues

3.6  SPH10 Resolved Issues

3.7  SPH7 Resolved Issues

3.8  SPH6 Resolved Issues

3.9  SPH3 Resolved Issues

3.10  SPH2 Resolved Issues

## SPH20 Resolved Issues

| Trouble Ticket Number | DTS2024062009109 |
| --- | --- |
| Description | For some HyperMetro LUNs, the host can scan only the paths to the storage system at one end of the HyperMetro pair. |
| Condition | 1. The Same Host LUN ID function is enabled (enabled by default) for HyperMetro. 2. Multiple LUNs are added to a LUN group. 3. There is a host LUN ID conflict between the two storage systems of the HyperMetro pair (local LUNs are configured at both ends of the HyperMetro pair). 4. The host is not an ESXi host. |
| Impact | The path of a HyperMetro LUN at one end cannot be reported. |
| Module | SCSI |
| Severity | Major |
| Root Cause | The automatic allocation logic is defective, failing to process the conflict. As a result, duplicate host LUN IDs are allocated in specific scenarios. |
| Solution | Update the allocation logic so that host LUN IDs are allocated by type instead of the read sequence. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024072323225 |
| --- | --- |
| Description | On DeviceManager, some controllers report alarms indicating that the CPU usage is excessively high. |
| Condition | When the host service load is light, background deduplication tasks run at a high speed. |
| Impact | On DeviceManager, controller CPU overload alarms are frequently reported. If an alarm monitoring platform is configured, alarms indicating that the CPU usage is excessively high are frequently received. |
| Module | Deduplication |
| Severity | Minor |
| Root Cause | Background deduplication tasks are running on the RV partition of the CPU. When deduplication tasks are running at a high speed, the usage of the RV partition increases, causing high CPU usage. |
| Solution | Optimize the overload alarm policy for the controller CPU usage in medium- and low-load scenarios. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024072001775 |
| --- | --- |
| Description | When SNMP is used to connect to a third-party NMS, the precise alarm mode is not supported. |
| Condition | The Nagios NMS software is connected. |
| Impact | The third-party NMS software cannot obtain the alarm severity through the MIB. |
| Module | SNMP |
| Severity | Minor |
| Root Cause | Storage systems using SNMP do not support the precise alarm mode. |
| Solution | Enable the storage systems to support the precise alarm mode. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024072208831 |
| --- | --- |
| Description | The reduction ratio is insufficient. |
| Condition | 1. Fingerprints are unevenly distributed. The number of fingerprints in a single DB exceeds 1.5 TB. 2. The write-ahead logs (WALs) of the opportunity table accounts for more than 90% of the total watermark of the opportunity table. |
| Impact | The reduction ratio is insufficient. |
| Module | Deduplication |
| Severity | Minor |
| Root Cause | The size of the manifest file cannot store all plog information, and services are severely uneven. |
| Solution | Increase the size of the manifest file. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024080107766 |
| --- | --- |
| Description | When an AD domain user and an NIS domain user with the same name are configured and mapping to users with the same name is set, the permissions are abnormal. |
| Condition | An AD domain user and an NIS domain user with the same name are configured and mapping to users with the same name is set. |
| Impact | The permissions of the AD domain user are abnormal. |
| Module | CIFS |
| Severity | Minor |
| Root Cause | After mapping to user with the same name is set, the permissions are not combined. |
| Solution | Query user permissions and mapping permissions and then combine the permissions. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024072406166 |
| --- | --- |
| Description | After synchronization is performed for NAS remote replication, the NFS share icon is missing. |
| Condition | In a remote replication scenario, dtrees and NFS shares are synchronized when the primary and secondary dtree IDs are inconsistent. |
| Impact | When dtrees are viewed on DeviceManager, the NFS share icon is not displayed. |
| Module | NFS |
| Severity | Minor |
| Root Cause | Querying dtree IDs using the NFS share alias fails. |
| Solution | Optimize the internal query logic. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024071119055 |
| --- | --- |
| Description | A controller resets when quotas are deleted. |
| Condition | 1. More than four template quotas are created. 2. The quotas are deleted. |
| Impact | A controller resets. |
| Module | File system |
| Severity | Major |
| Root Cause | When the number of template quotas exceeds 4, an out-of-bounds array access occurs when the quotas are deleted. As a result, the controller resets. |
| Solution | Mount the created template quotas to the linked list to prevent out-of-bounds array access. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024071119408 |
| --- | --- |
| Description | The latency is high due to writes after truncation. |
| Condition | The same file is repeatedly copied to the same directory of a file system. |
| Impact | The performance deteriorates. |
| Module | Filesystem |
| Severity | Major |
| Root Cause | When dirty data exists in the area where data is to be written, reclamation is triggered. The reclamation is processed in serial mode, which increases the latency. |
| Solution | Reclaim dirty data in concurrent mode. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024081705430 |
| --- | --- |
| Description | The storage system reports a file system media error alarm. |
| Condition | During incremental synchronization for a file system HyperMetro pair, large files are truncated when the synchronization is about to complete. |
| Impact | The storage system reports a file system media error alarm. |
| Module | Volume |
| Severity | Major |
| Root Cause | During the incremental synchronization for a file system HyperMetro pair, the internal parity bits are incorrectly marked when files are truncated. As a result, an alarm is reported. |
| Solution | Resolve the problem that parity bits are incorrectly marked when large files are truncated during incremental synchronization for HyperMetro pairs to prevent alarms from being reported. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024071600671 |
| --- | --- |
| Description | Ports on Ethernet interface modules are intermittently disconnected. |
| Condition | A switch is connected to multiple (more than 25) bond offload ports of Ethernet interface modules. |
| Impact | Ports on Ethernet interface modules are intermittently disconnected. |
| Module | LACP |
| Severity | Major |
| Root Cause | The LACP packet exchange of the bond offload ports on the Ethernet interface modules is too fast, exceeding the LACP packet rate limit (128 by default) of the switch. As a result, the switch and storage ports are intermittently disconnected. |
| Solution | Optimize the LACP packet exchange frequency of Ethernet interface modules to keep it below the LACP packet rate limit of the switch. |
| Solution Impact | None |

## SPH18 Resolved Issues

| Trouble Ticket Number | DTS2023121303231 |
| --- | --- |
| Description | Data erasing is selected when a disk domain is deleted, but the disk fails to be connected to the storage system after data is erased. |
| Condition | 1. The disk is an NVMe self-encrypting disk whose firmware version is 8217. 2. A data erasing task is executed. |
| Impact | The disk fails to be connected to the storage system after data destruction. |
| Module | BDM |
| Severity | Major |
| Root Cause | There is a low probability that a time sequence error occurs during protocol negotiation with the disk. As a result, command conflicts occur, the disk destruction process fails, and disk access authentication fails. |
| Solution | Solve the time sequence problem in protocol negotiation with disks. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024060702995 |
| --- | --- |
| Description | An LDAP user fails to log in to DeviceManager. |
| Condition | The user name or password of an LDAP user account contains special characters, for example, French characters é (inconsistency between UTF-8 and UTF-16 encoding formats). |
| Impact | Logging in to DeviceManager fails. |
| Module | DeviceManager |
| Severity | Major |
| Root Cause | The character encoding format is not converted. |
| Solution | Convert the content to the UTF-8 format before delivery. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024022612148 |
| --- | --- |
| Description | Although Do not show this message again is selected in the Confirm Display dialog box on DeviceManager, the setting does not take effect. |
| Condition | The zoom of the browser or operating system is not 100%. |
| Impact | This dialog box is displayed each time a user re-logs in to DeviceManager after logging out of DeviceManager or closing the browser. |
| Module | DeviceManager |
| Severity | Minor |
| Root Cause | The tag of the Do not show this message again option is placed in the cookie and the setting will be cleared after a user logs out of DeviceManager. |
| Solution | Store the tag in the browser cache. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024061328119 |
| --- | --- |
| Description | A CIFS share fails to be mounted using an FQDN. |
| Condition | 1. vStores are added to the AD domain. 2. Container services are enabled. |
| Impact | There is a possibility that the CIFS share fails to be mounted. |
| Module | Devm |
| Severity | Major |
| Root Cause | The protocol proactively pushes the internal IP address of the container to the DNS server.During domain name resolution, there is a possibility that the internal IP address of the container is returned. |
| Solution | Optimize the system to disable the proactive push mechanism. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024060604755 |
| --- | --- |
| Description | The system reports a component exception alarm. |
| Condition | The deduplication service has been configured for the storage system and the version is being upgraded to 6.1.6. The pre_merge operation is interrupted when app_data is restarted during the upgrade. |
| Impact | When the service data volume is large, the data reduction rate decreases. In addition, an alarm is displayed on DeviceManager eight days after the upgrade. |
| Module | Deduplication |
| Severity | Major |
| Root Cause | The fingerprint sequence numbers are not persistently stored, and fixed-length blocks are repeatedly written back to the DB. As a result, the internal deduplication process becomes slow. |
| Solution | Use the persistent sequence numbers in the DB and determine whether to write fixed-length blocks back to the DB based on the current DB size. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024061502478 |
| --- | --- |
| Description | When a file or directory is created in an NFSv3 share, a message is displayed indicating that the file name is too long. |
| Condition | When multiple languages are configured for NFSv3, the name of a created file or directory exceeds 255 characters after being converted to UTF-8. |
| Impact | The file cannot be created. |
| Module | NFS |
| Severity | Minor |
| Root Cause | After being converted to UTF-8, the name length exceeds 255 characters, which does not meet the internal judgment logic. |
| Solution | Optimize the internal judgment logic. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024061328869 |
| --- | --- |
| Description | Users cannot access the Homedir share. |
| Condition | When accessing the Homedir share, the user also changes the UTC time of the system or performs NTP time synchronization. |
| Impact | The Homedir share cannot be accessed. |
| Module | CIFS |
| Severity | Major |
| Root Cause | The of the system is changed to a value earlier than the previous one, the timeout calculation is reversed and the system incorrectly determines that the access times out. |
| Solution | Optimize the timeout determination process. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024032507675 |
| --- | --- |
| Description | Capacity statistical items that cannot be identified by VMs exist in the VMware datastore capacity statistics. |
| Condition | 1. VAAI is enabled during file system creation. 2. The NFS share is mounted to VMware to create a datastore. 3. VMs migration are repeatedly perfoemed in the datastore. |
| Impact | The capacity display is affected. |
| Module | Filesystem |
| Severity | Minor |
| Root Cause | When the service load is heavy and VMs are migrated repeatedly, the bitmap statistics of the storage file system are inaccurate. |
| Solution | Optimize the bitmap statistics logic. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2023091219466 |
| --- | --- |
| Description | The queried capacity of a HyperMetro or remote replication pair is incorrect. |
| Condition | After a file system is created and then a HyperMetro or remote replication pair is created for the file system, capacity of the HyperMetro or remote replication pair is queried. |
| Impact | The queried HyperMetro or remote replication pair capacity of the file system is incorrect. |
| Module | Filesystem |
| Severity | Minor |
| Root Cause | The queried file system capacity should be in the unit of sector instead of byte. |
| Solution | Change the unit to sector. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024070505526 |
| --- | --- |
| Description | OpenSSH has a level-1 vulnerability [HWPSIRT-2024-56267][CVE-2024-6387]. |
| Condition | The loginGraceTIme function is enabled. |
| Impact | Attackers can exploit this vulnerability to execute code with the root permission, which poses security risks. |
| Module | OpenSSH |
| Severity | Major |
| Root Cause | The signal handler race condition in the OpenSSH server (sshd) can be maliciously used. |
| Solution | Use the rectification solution provided by the OpenSSH open-source community. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024051400127 |
| --- | --- |
| Description | When links are unstable, resource leak occurs on a storage port. As a result, host services carried on the port cannot be delivered properly. |
| Condition | Necessary conditions: 1. On a SAN switched network, a host port in the zone experiences repeated intermittent disconnection. 2. During connection setup, the front-end host port does not initiate a PLOGI connection setup request according to the protocol. As a result, the driver cannot offload the connection information to the chip. 3. The intermittently disconnected host port in the zone does not respond to the LOGO frame sent by the storage system. There is a possibility that the problem occurs when conditions 1, 2, and 3 are met. |
| Impact | A chip resource leak occurs, and services may be interrupted. |
| Module | FC |
| Severity | Major |
| Root Cause | Resources are not restored in a timely manner. |
| Solution | Optimize the resource application and release in the case of repeated intermittent link disconnection. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024031926434 |
| --- | --- |
| Description | After the NAS backup write service is started, the service performance deteriorates. |
| Condition | 1. The DTOE function is enabled on the storage device. 2. A large number of packets are lost due to network congestion or transient heavy service pressure. |
| Impact | The host bandwidth decreases. In severe cases, the host bandwidth decreases to zero intermittently. |
| Module | DTOE |
| Severity | Major |
| Root Cause | When a large number of packets are lost, the storage TCP window resources are not adjusted in a timely manner. As a result, the receiving efficiency decreases and the host performance deteriorates. |
| Solution | Optimize the policy for processing storage TCP window resources in fault scenarios. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024070216930 |
| --- | --- |
| Description | Controllers reset unexpectedly. |
| Condition | The device is added to DME and performance data collection has been running for a long time. |
| Impact | Controllers reset unexpectedly. |
| Module | OMM |
| Severity | Major |
| Root Cause | Memory leak occurs on a download interface. |
| Solution | Optimize the memory release process. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024060502223 |
| --- | --- |
| Description | Clients cannot access an NFS share. |
| Condition | A large number of clients access the storage system. Some clients deliver service I/Os at an interval of more than 3 hours. |
| Impact | Clients cannot access the NFS share. |
| Module | NFS |
| Severity | Major |
| Root Cause | Duplicate hash values of client IP addresses are identified. The memory space is not released in a timely manner during background clearance. |
| Solution | Optimize the background clearing mechanism. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024070506532 |
| --- | --- |
| Description | After hosts are powered off, the alarm indicating that the number of concurrent NAS connections reaches 95% of the upper limit persists on DeviceManager. |
| Condition | Concurrent connections reach the specifications and a large number of hosts are powered off at the same time. |
| Impact | Services are interrupted. |
| Module | NFS&CIFS |
| Severity | Major |
| Root Cause | After the hosts are powered off, they cannot send acknowledge messages to the storage system. As a result, the concurrency resources fail to be released. |
| Solution | Optimize the release logic for internal concurrency resources. |
| Solution Impact | None |

## SPH17 Resolved Issues

| Trouble Ticket Number | DTS2024061423976 |
| --- | --- |
| Description | The CPU usage of storage controllers is high, the performance is affected, and the HyperMetro pair is disconnected. |
| Condition | 1. Real-time ransomware detection is enabled. 2. The file system for which ransomware detection is enabled has tens of thousands of directories and files and has many directory levels. 3. Directories are renamed and the files in a directory are modified. |
| Impact | The I/O performance deteriorates. |
| Module | File system |
| Severity | Major |
| Root Cause | A snapshot comparison scanning task is deleted before it is complete, and there is a low probability that thread resources are occupied for a long time. |
| Solution | Optimize the background scanning process after a snapshot comparison task is abnormally deleted. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024061415076 |
| --- | --- |
| Description | In NAS file sharing scenarios, if the same file is copied to a directory for multiple times, there is a low probability that read and write exceptions occur. |
| Condition | The same file is repeatedly copied to a directory. |
| Impact | There is a low probability that read and write exceptions occur. |
| Module | File system |
| Severity | Major |
| Root Cause | In overwrite scenarios, there is a low probability that the data writes in the file system and background reclamation tasks are not mutually exclusive. |
| Solution | Resolve the issue that write I/Os and background reclamation tasks are not mutually exclusive in overwrite scenarios. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024061800232 |
| --- | --- |
| Description | After the device runs for a long time, the memory of the smart disk enclosure and controller enclosure is insufficient. As a result, the OOM reset occurs. |
| Condition | The device with a kernel version of 1.1.1.0 has been running for a long time. |
| Impact | Services are interrupted. |
| Module | SWM |
| Severity | Major |
| Root Cause | The threading module of the scheduled task for synchronizing logs using a Python script has a defect. As a result, memory leakage occurs when tasks are executed. |
| Solution | Optimize the scheduled task. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024061516304 |
| --- | --- |
| Description | Users cannot access the Homedir share. |
| Condition | The automatic path creation function is not enabled when the Homedir share mapping rule is added, and a directory exists in the share path of the mapping rule. The directory does not fully match the relative path due to case sensitiveness. |
| Impact | Users cannot access the Homedir share. |
| Module | CIFS |
| Severity | Major |
| Root Cause | When the relative path is matched for Homedir share access, a case-sensitive policy is adopted. As a result, the relative path does not completely match the directory path in the file system path, and the path cannot be found. |
| Solution | Use a case-insensitive policy when the relative path is matched for Homedir share access. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024061710799 |
| --- | --- |
| Description | The storage system reports the alarm of "An Exception Occurred During Intelligent Ransomware Detection". |
| Condition | 1. Intelligent ransomware detection is configured. 2. After the previous ransomware detection, the file content is modified and the directory where the file is located is changed. |
| Impact | Snapshot detection fails and the storage system reports the alarm. |
| Module | HyperDetect |
| Severity | Minor |
| Root Cause | After the file and its parent directory are modified, the file operation type is defined as modification. During ransomware detection, the file is compared with that in the last snapshot. However, the detection fails because the path is changed. |
| Solution | Upgrade the ransomware protection software to 1.3.0.SPC17. |
| Solution Impact | None |

## SPH15 Resolved Issues

| Trouble Ticket Number | DTS2024052218319 |
| --- | --- |
| Description | There is a low probability that I/Os suspend after a process reset. |
| Condition | After a clone pair or snapshot is deleted, a version upgrade is performed or a process is reset. |
| Impact | Services are interrupted. |
| Module | Volume |
| Severity | Major |
| Root Cause | When a clone pair or snapshot is deleted, the LUN relationship attributes are retained in the memory. After the upgrade or process reset, some attributes are not restored. |
| Solution | Optimize the process of deleting clone pairs and snapshots. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024052514709 |
| --- | --- |
| Description | In the SAN 3DC scenario, storage is not optimized for adapting to certain service models. As a result, an error is reported during database archiving. |
| Condition | This issue occurs in certain service models. |
| Impact | Errors occur during database archiving. |
| Module | Replication |
| Severity | Major |
| Root Cause | In specific service scenarios, the storage is not optimized for service adaption. As a result, an error is reported during database archiving. |
| Solution | Optimize the I/O process in specific service scenarios for adaption. |
| Solution Impact | None |

## SPH13 Resolved Issues

| Trouble Ticket Number | DTS2024050300091 |
| --- | --- |
| Description | The VM service I/O write latency of the file system is high. |
| Condition | (1) The VAAI function is enabled for the file system. (2) The file system is mounted to VMware. (3) Batch VM cloning is performed for the mounted file system in heavy-load scenarios. |
| Impact | The VM service write I/O latency of the file system is high. |
| Module | VAAI |
| Severity | Major |
| Root Cause | The VAAI bitmap update is slow. As a result, the latency of VM write services is long. |
| Solution | 1. Change the bitmap queue depth to 4,000,000. 2. Optimize the timeout interval. |
| Solution Impact | None |

## SPH10 Resolved Issues

| Trouble Ticket Number | DTS2024041122201 |
| --- | --- |
| Description | The controller enclosure is powered off unexpectedly. |
| Condition | The capacitor of the power-off switch circuit on the controller enclosure fails (this occurs with an extremely low probability), pulling down the power-off control level. |
| Impact | The controller enclosure is powered off unexpectedly, and services are interrupted. |
| Module | DMI |
| Severity | Major |
| Root Cause | According to the current power-off policy, the controller enclosure will be powered off when the capacitor fails. |
| Solution | Disable the function of powering off the device by pressing the power button by default. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024032114544 |
| --- | --- |
| Description | Fault detection and alarm policies for silent corruptions of the system disk are optimized. |
| Condition | The system disk experiences verification errors. |
| Impact | No internal recovery mechanism is available if a system disk of a controller is faulty. |
| Module | FDSA |
| Severity | Major |
| Root Cause | Silent corruptions occur in some partitions and files of the system disk. However, there is no detection on these faults and no alarm is reported. |
| Solution | Add detection and recovery measures for verification errors in key partitions of the system disk. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024011008336 |
| --- | --- |
| Description | The remote device of a remote replication pair is disconnected after an upgrade. |
| Condition | A version earlier than 6.1.2 is upgraded to 6.1.2 or a later version, and a remote device with an IP link is configured. |
| Impact | The remote device is disconnected after the upgrade. |
| Module | Replication |
| Severity | Minor |
| Root Cause | The link ID is incorrectly processed during the upgrade. |
| Solution | Optimize the link ID processing policy during the upgrade to prevent the remote device from being affected. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024040202677 |
| --- | --- |
| Description | A timeout error is reported when the 32 GB bitmap is queried. |
| Condition | Large-granularity bitmap query is performed when the service load is heavy |
| Impact | The bitmap fails to be queried using the REST interface. As a result, the CV backup becomes slow. |
| Module | Volume |
| Severity | Major |
| Root Cause | The system splits bitmap requests based on 256 KB and then performs serial query. As a result, the bitmap request processing time exceeds the timeout threshold. |
| Solution | Return the queried result immediately when a large-granularity bitmap query is about to time out. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024022819517 |
| --- | --- |
| Description | The kernel version fails to be upgraded, and services are interrupted. |
| Condition | A large number of full copy services are delivered to reach the maximum specifications and a node fault occurs. |
| Impact | Services are interrupted. |
| Module | Volume |
| Severity | Major |
| Root Cause | After a node is faulty, the battery-protected memory quota changes. As a result, the watermark calculation is abnormal, and the system process cannot run properly. |
| Solution | Recalculate the watermark if the watermark calculation is abnormal during node fault handling. This ensures the normal running of process. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024041103334 |
| --- | --- |
| Description | Snapshot reclamation is slow. |
| Condition | The last snapshot or CDP is deleted from the file system. |
| Impact | The protection capacity reclamation is slow. |
| Module | Volume |
| Severity | Minor |
| Root Cause | When a snapshot is deleted from a file system, full scanning and reclamation are triggered by mistake. As a result, it takes a long time to reclaim the capacity used for data protection. |
| Solution | Add a condition for determining the protection capacity to ensure that full reclamation is started only when the protection capacity is not 0. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024040908937 |
| --- | --- |
| Description | Logging in to ProtectManager fails. |
| Condition | BGP services and ransomware protection or cloud backup services are deployed in high-end environments. |
| Impact | The ProtectManager interface cannot be logged in to. |
| Module | Devm |
| Severity | Major |
| Root Cause | When the high-end storage container processes the login request, the source IP address of the packet sent during internal communication of the container conflicts with the BGP VIP on the loopback interface. As a result, the response packet cannot be received, and the login request times out. |
| Solution | Add a VIP local route. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024032809862 |
| --- | --- |
| Description | An exception occurs when host services access storage LUNs. As a result, the latency is high or disks are offline. |
| Condition | A self-encrypting disk domain is created for self-encrypting disks. When the power supply is faulty and the voltage is unstable, current backfeed occurs. |
| Impact | Services are interrupted. |
| Module | BDM |
| Severity | Major |
| Root Cause | When a disk enclosure experiences a power failure, current backfeed occurs on the power supply. After the power supply is recovered, the self-encrypting disks fail to be authenticated as expected. As a result, an I/O error is reported when the storage disks are accessed. |
| Solution | Correct the logic for processing self-encrypting disks in this scenario to resolve the problem of unauthenticated disks. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024032809795 |
| --- | --- |
| Description | The system disk reports a slow disk alarm. |
| Condition | The disk flushing latency of the system disk is long. |
| Impact | The system disk reports fault alarms. |
| Module | BDM |
| Severity | Major |
| Root Cause | When the read/write load of the system disk is high, the disk flushing latency increases, reaching the alarm threshold of slow disk detection. |
| Solution | Optimize the slow-disk detection standard. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024010910250 |
| --- | --- |
| Description | When a plug-in is connected to the storage device and a large number of VMs are provisioned concurrently, there is a possibility that disks cannot be attached to some VMs. |
| Condition | A plug-in is connected to the storage device and a large number of VMs are provisioned concurrently. |
| Impact | There is a possibility that disks cannot be attached to VMs during VM provisioning using a plug-in. |
| Module | CFG |
| Severity | Major |
| Root Cause | A large number of iSCSI target query requests are delivered to the storage system, exceeding its execution capability. A message is displayed indicating that the system is busy. |
| Solution | Optimize the iSCSI target query process to improve the interface performance. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024032115284 |
| --- | --- |
| Description | A performance breakpoint occurs on the DME GUI. |
| Condition | DME is connected to an 8-controller storage system that has more than eight smart disk enclosures. |
| Impact | There is a possibility that performance breakpoints occur on the DME GUI. |
| Module | Perf |
| Severity | Minor |
| Root Cause | Real-time collection tasks are not processed in a timely manner, and performance data breakpoints occur. |
| Solution | Optimize the real-time collection task process. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024042312536 |
| --- | --- |
| Description | A domain authentication user cannot log in to the storage device. |
| Condition | The actual name of the domain authentication user is different from its samAccountName on the domain authentication server. |
| Impact | The domain authentication user cannot log in to the storage device. |
| Module | AA |
| Severity | Minor |
| Root Cause | The storage system enhances the verification of domain authentication user names. As a result, some domain authentication users cannot log in to the storage system. |
| Solution | Optimize the logic for verifying domain authentication users. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024012509790 |
| --- | --- |
| Description | After a BGP peer creation fails, an error message is displayed, indicating that the system is busy. |
| Condition | After the BGP peer is configured, it remains in the Connect state due to incorrect configurations, wrong cable connections, or network disconnection. |
| Impact | The BGP peer cannot be created. |
| Module | BGP |
| Severity | Minor |
| Root Cause | The resources applied for the TCP connection are not released in a timely manner after the BGP peer connection fails. |
| Solution | Optimize the mechanism for releasing resources after connection failures of the BGP peer. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024041507691 |
| --- | --- |
| Description | The Snapshot Comparison function suspends when it is invoked by HyperDetect to compare snapshot differences. |
| Condition | (1) File systems of a HyperMetro pair are created. (2) A logical port that belongs to the secondary end of the HyperMetro pair is created and used for mounting. (3) A dtree is created and directories in the dtree are renamed. (4) An FSI scanning task is created or the HyperDetect feature is used to invoke the Snapshot Comparison function. |
| Impact | The Snapshot Comparison function fails. |
| Module | Snapshot Comparison |
| Severity | Major |
| Root Cause | When the Snapshot Comparison function is invoked in the HyperMetro cluster, it is not switched to the corresponding HyperMetro pair. |
| Solution | Switch the Snapshot Comparison function to the corresponding HyperMetro pair when it is invoked in a HyperMetro cluster. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024030511147 |
| --- | --- |
| Description | The system disk wears out too fast. |
| Condition | The system disk has residual dirty data of production tests for a long time. |
| Impact | The wear of system disk is aggravated, shortening its service life. |
| Module | Equipment |
| Severity | Minor |
| Root Cause | Residual dirty data after production tests causes the available space of the system disk to decrease, accelerating the wear of the writable space. |
| Solution | Clear dirty data from the system disk. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024031404215 |
| --- | --- |
| Description | The write latency increases temporarily after write data is appended to the truncated file. |
| Condition | After truncating a file, append data to the file immediately. |
| Impact | The write latency of the truncated file increases temporarily. |
| Module | Filesystem |
| Severity | Minor |
| Root Cause | The background does not reclaim the file data after the file is truncated. In this case, when a write request is delivered to the file, a write lock is added to the file, triggering data reclamation. As a result, a lock conflict occurs. |
| Solution | Change the lock scheme of write requests after file truncation to range write lock to reduce lock conflicts. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2023122306508 |
| --- | --- |
| Description | A controller resets unexpectedly when services are running on the storage system. |
| Condition | The service load is heavy. |
| Impact | A controller resets unexpectedly. |
| Module | BIOS |
| Severity | Major |
| Root Cause | The CPU chip configuration is improper. As a result, the number of concurrent requests of the extension chip exceeds the total number of concurrent requests of the CPU. When the service load is heavy, a process deadlock may occur on the CPU, causing the controller to reset unexpectedly. |
| Solution | Optimize the chip concurrency control logic to prevent process deadlocks. After the patch is installed, reset the controller for the modification to take effect. |
| Solution Impact | None |

## SPH7 Resolved Issues

| Trouble Ticket Number | DTS2024021702091 |
| --- | --- |
| Description | There is a low probability that the system upgrade fails when a user name with non-UTF-8 encoded characters in the LDAP group is used to logs in to the storage system. |
| Condition | A user name with non-UTF-8 encoded characters in the LDAP group is used to logs in to the storage system. |
| Impact | The system upgrade fails. |
| Module | AA |
| Severity | Major |
| Root Cause | The LDAP group contains user names with non-UTF-8 encoded characters. The file opening verification fails. |
| Solution | Optimize the file opening logic. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024021702137 |
| --- | --- |
| Description | The storage system reports an alarm indicating that the NTP service is unavailable. |
| Condition | The IPv6 address configured for the NTP domain name of the DNS server contains more than 40 characters. The NTP server is added to the storage system using the domain name. |
| Impact | The time cannot be synchronized with the NTP server. |
| Module | NTP |
| Severity | Major |
| Root Cause | The IPv6 address after domain name resolution exceeds the internal length limit. As a result, the NTP service restarts repeatedly. |
| Solution | Optimize the IP address conversion logic. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024022203865 |
| --- | --- |
| Description | Storage pool creation fails. |
| Condition | A storage pool is created using more than 100 disks. |
| Impact | Storage pool creation fails. |
| Module | Pool |
| Severity | Minor |
| Root Cause | When more than 100 disks are used to create a storage pool, the calculation of the verification function is defective. As a result, the storage pool creation fails. |
| Solution | Rectify the calculation defect when more than 100 disks are used to create a storage pool. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024020515761 |
| --- | --- |
| Description | After the device is powered on, the rotational speed of fans is intermittently high. |
| Condition | a. 1822 V120 interface modules are configured. b. The ambient temperature is between 15°C and 25°C. c. The service load is low. There is a possibility that this issue occurs when conditions a, b, and c are met. |
| Impact | The noise of fans fluctuates repeatedly. |
| Module | Pangea |
| Severity | Minor |
| Root Cause | When the temperature of interface modules increases by 3°C, the fan speed level increases from 8 to 20. As a result, the fan speed is frequently adjusted when the temperature increases or decreases. |
| Solution | Optimize the fan speed level adjustment. The start fan speed level should be manually adjusted to reduce the frequency of triggering fan speed level adjustment. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024020508255 |
| --- | --- |
| Description | After a controller is removed and re-inserted, an alarm indicating that the interface module fails to be powered on is generated on the controller. |
| Condition | 1. The Hi1822 V120 high-end interconnect I/O module is used. 2. The controller is removed and re-inserted. There is a high probability that this issue occurs when the preceding conditions are met. |
| Impact | An alarm on the interface module is generated and ports on the module are unavailable. |
| Module | MPU |
| Severity | Major |
| Root Cause | When a packet Completion Time-Out (CTO) event occurs, the corresponding bit of the register is not cleared. As a result, packet CTO events continuously reported and the driver probe fails to started for multiple times, making ports unavailable. |
| Solution | Clear the corresponding bit of the register in the interrupt handling process. |
| Solution Impact | None |

## SPH6 Resolved Issues

| Description | In some scenarios, the deduplication service stops unexpectedly due to a plog deletion failure. |
| --- | --- |
| Severity | Major |
| Root Cause | In some scenarios, a plog deletion process terminates unexpectedly. After three retries, the deletion still fails. As a result, the deduplication service stops. |
| Solution | Add a procedure for handling the abnormal termination of plog deletions to prevent the deduplication service from being stopped abnormally. |
| Solution Impact | None |

| Description | In some service overload scenarios, the deduplication process is oversuppressed, affecting the deduplication efficiency. |
| --- | --- |
| Severity | Minor |
| Root Cause | The deduplication rate is over reduced by flow control in overload scenarios. |
| Solution | Optimize the deduplication flow control logic to improve deduplication efficiency. |
| Solution Impact | None |

| Description | Recording audit logs in XML format fails. |
| --- | --- |
| Severity | Minor |
| Root Cause | The files name contains escape characters and non-UTF-8 characters. As a result, parsing the file as an UTF-8 file fails. |
| Solution | Optimize the parsing logic. |
| Solution Impact | None |

| Description | The write I/O latency increases during fullcopy. |
| --- | --- |
| Severity | Minor |
| Root Cause | During fullcopy, I/Os are unevenly distributed. As a result, the CPU usage of some cores surges and the latency increases. |
| Solution | Optimize the I/O distribution rules. |
| Solution Impact | None |

| Description | After the system is upgraded to 6.1.6, a license error is returned when a thick LUN is created using the RESTful API. |
| --- | --- |
| Severity | Minor |
| Root Cause | In 6.1.6, the return value of the RESTful API for creating volumes is changed. As a result, an adaptation problem occurs. |
| Solution | Optimize the return value logic of RESTful APIs. |
| Solution Impact | None |

| Description | An alarm is reported indicating that the coffer_log directory usage exceeds the threshold because there are too many GaussDB component logs. |
| --- | --- |
| Severity | Minor |
| Root Cause | Historical logs are not deleted because the log management process is abnormal. |
| Solution | Add a daemon process to the log management process to ensure that logs can be properly deleted. |
| Solution Impact | None |

| Description | The capacity statistics of HyperMetro multi-dtree file systems in the config log is incorrect. |
| --- | --- |
| Severity | Minor |
| Root Cause | The capacity statistics of the show file_system reduction command output is not correct in HyperMetro file system scenarios. |
| Solution | Optimize the capacity statistics of the show file_system reduction command in HyperMetro file system scenarios. |
| Solution Impact | None |

| Description | After the NAS client sends a NullCall packet to port 2052 of the storage device, the storage device returns a NullReply packet. |
| --- | --- |
| Severity | Minor |
| Root Cause | When processing packets, the storage device does not correctly determine the relationship between packet requests and service ports. |
| Solution | Correct the determination logic so that the relationship between packet requests and service ports can be correctly determined. |
| Solution Impact | None |

| Description | The read and write performance of the storage system deteriorates because the client repeatedly establishes and disconnects TCP connections to and from the storage system. |
| --- | --- |
| Severity | Minor |
| Root Cause | Internal resources are not correctly released after disconnections. As a result, the I/O latency increases after a long period of time. |
| Solution | Optimize the process of releasing resources after disconnection. |
| Solution Impact | None |

| Description | When multiple clients concurrently lock a file, there is a possibility that the NLM lock status on the host side differs from that on the storage side. |
| --- | --- |
| Severity | Minor |
| Root Cause | When the NLM lock asynchronous notification connection between the storage device and the client fails, the connection is retried for a long time. Notification messages from other hosts are blocked. As a result, the lock status on the host side is inconsistent with that on the storage side. |
| Solution | Optimize the processing logic when the NAS client delivers the cancel command word and enable the processing logic after the NLM lock notification connection fails by default. |
| Solution Impact | None |

| Description | Alarms to be cleared are frequently sent in the Air Gap scenario. |
| --- | --- |
| Severity | Minor |
| Root Cause | In the Air Gap scenario, alarms to be cleared on the primary storage are not suppressed. |
| Solution | Optimize the alarm suppression policy of the primary storage in the Air Gap scenario. |
| Solution Impact | None |

| Description | The space is not reclaimed after files are deleted from a file system. |
| --- | --- |
| Severity | Major |
| Root Cause | The mapping between front-end and back-end shards is not initialized before data is written. |
| Solution | Write data only after the mapping between front-end and back-end shards is initialized. |
| Solution Impact | None |

## SPH3 Resolved Issues

| Description | The deduplication efficiency needs to be optimized. |
| --- | --- |
| Severity | Minor |
| Root Cause | Due to the large size of the deduplication opportunity table, the deduplication efficiency does not meet the expectation. |
| Solution | Reduce the size of the deduplication opportunity table by optimizing the deduplication stock tree. In addition, commands are provided to manually adjust the maximum capacity of the deduplication opportunity table. |
| Solution Impact | None |

| Description | Latency increases because back-end acceleration resources are not released in a timely manner and the service pressure approaches the back-end bottleneck. |
| --- | --- |
| Severity | Major |
| Root Cause | Back-end acceleration resources are not released in a timely manner, and the service pressure approaches the back-end bottleneck. As a result, the latency increases. |
| Solution | Solve the problem of delayed releasing of back-end acceleration resources. |
| Solution Impact | None |

| Description | After the storage system is upgraded to 6.1.6, an alarm indicating that the capacity is used up is generated, and host services are abnormal due to a space request failure. |
| --- | --- |
| Severity | Major |
| Root Cause | In certain scenarios, a reserved field of the storage pool is not correctly processed. After the upgrade, the system uses the value of this field and incorrectly determines that the storage pool capacity is used up. As a result, new space cannot be allocated to host services. |
| Solution | Correct the processing logic and the storage pool capacity. |
| Solution Impact | None |

## SPH2 Resolved Issues

| Description | The VM migration delay is long, affecting service performance. |
| --- | --- |
| Severity | Minor |
| Root Cause | In the deduplication process, there is a large cycle of fingerprint comparison, affecting CPU scheduling and prolonging service latency. |
| Solution | Optimize the transition period of the deduplication process. |
| Solution Impact | None |

| Description | The link for connecting to FalconStor virtualization is faulty. |
| --- | --- |
| Severity | Major |
| Root Cause | A compatibility issue occurs when a command word is queried because the identifying field is XSG but not VAC. |
| Solution | Change the identifying field of the command word from XSG to VAC. |
| Solution Impact | None |

| Description | Directory redirection fails after the offline cache function is enabled for a share. |
| --- | --- |
| Severity | Minor |
| Root Cause | The permission bit in some request packets is abnormal. As a result, the client does not have the permission. |
| Solution | Optimize permission bit processing in packets. |
| Solution Impact | None |

| Description | The long-distance network for RDMA-based replication has high transmission latency in large I/O scenarios. |
| --- | --- |
| Severity | Minor |
| Root Cause | In long-distance RDMA-based replication scenarios, the policy for processing large I/Os is inappropriate. As a result, the transmission latency increases. |
| Solution | Optimize the policy for processing large I/Os in long-distance RDMA-based replication scenarios. |
| Solution Impact | None |

# Third-Party Software Vulnerability Fixes

Vulnerability information can be queried using CVE IDs on the National Vulnerability Database (NVD) website: http://web.nvd.nist.gov/view/vuln/search.

Third-party software vulnerability fixes

| Software | Version | CVE ID | Vulnerability Description | Solution |
| --- | --- | --- | --- | --- |
| OpenSSH | openssh-8.8p1-2.h19.eulerosv2r11 8.8p1-h5 | CVE-2024-6387 | A security regression (CVE-2006-5051) was discovered in OpenSSH's server (sshd). There is a race condition which can lead to sshd to handle some signals in an unsafe manner. An unauthenticated, remote attacker may be able to trigger it by failing to authenticate within a set time period. | Upgrade to openssh-8.8p1-2.h34.eulerosv2r11 Upgrade to 8.8p1-18.oe2203sp1-htrunk20 |