|  | OceanStor V5 Series Storage Systems V500R007C71SPH152 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Patch Notes |  |  |  |
|  | Issue | 01 |  |  |
|  | Date | 2024-05-15 |  |  |
|  | HUAWEI TECHNOLOGIES CO., LTD. |  |  |  |

Copyright © Huawei Technologies Co., Ltd. 2024. All rights reserved. No part of this document may be reproduced or transmitted in any form or by any means without prior written consent of Huawei Technologies Co., Ltd. Trademarks and Permissions and other Huawei trademarks are trademarks of Huawei Technologies Co., Ltd. All other trademarks and trade names mentioned in this document are the property of their respective holders. Notice The purchased products, services and features are stipulated by the contract made between Huawei and the customer. All or part of the products, services and features described in this document may not be within the purchase scope or the usage scope. Unless otherwise specified in the contract, all statements, information, and recommendations in this document are provided "AS IS" without warranties, guarantees or representations of any kind, either express or implied. The information in this document is subject to change without notice. Every effort has been made in the preparation of this document to ensure accuracy of the contents, but all statements, information, and recommendations in this document do not constitute a warranty of any kind, express or implied.

| Huawei Technologies Co., Ltd. |  |
| --- | --- |
| Address: | Huawei Industrial Base Bantian, Longgang Shenzhen 518129 People's Republic of China |
| Website: | https://e.huawei.com |

Security Declaration

Product Lifecycle Huawei's regulations on product lifecycle are subject to the Product End of Life Policy. For details about this policy, visit the following web page: https://support.huawei.com/ecolumnsweb/en/warranty-policy Vulnerability Huawei's regulations on product vulnerability management are subject to the Vul. Response Process. For details about this process, visit the following web page: https://www.huawei.com/en/psirt/vul-response-process For vulnerability information, enterprise customers can visit the following web page: https://securitybulletin.huawei.com/enterprise/en/security-advisory Preconfigured Digital Certificate The digital certificates preconfigured on Huawei devices are subject to the Rights and Responsibilities of Preconfigured Digital Certificates on Huawei Devices. For details about this document, visit the following web page: https://support.huawei.com/enterprise/en/bulletins-service/ENEWS2000015789 Huawei Enterprise End User License Agreement This agreement is the end user license agreement between you (an individual, company, or any other entity) and Huawei for the use of the Huawei Software. Your use of the Huawei Software will be deemed as your acceptance of the terms mentioned in this agreement. For details about this agreement, visit the following web page: https://e.huawei.com/en/about/eula Lifecycle of Product Documentation Huawei after-sales user documentation is subject to the Product Documentation Lifecycle Policy. For details about this policy, visit the following web page: https://support.huawei.com/enterprise/en/bulletins-website/ENEWS2000017761

Change History

| Issue | Date | Description |
| --- | --- | --- |
| 01 | 2024-05-15 | This issue is the first official release. |

Contents

Change History	iii

1 Patch Overview	1

1.1 Basic Information	1

1.2 Impact of Patch Installation	2

1.2.1 Impact on the System During Installation	2

1.2.2 Impact on the System After Installation	2

1.3 Version Compatibility Requirements	2

2 Resolved Issues	3

2.1 V500R007C71SPH152	3

2.2 V500R007C71SPH150	4

2.3 V500R007C71SPH137	7

2.4 V500R007C71SPH136	8

2.5 V500R007C71SPH132	11

2.6 V500R007C71SPH130	12

2.7 V500R007C71SPH120	15

2.8 V500R007C71SPH115	17

2.9 V500R007C71SPH112	19

2.10 V500R007C71SPH111	20

2.11 V500R007C71SPH107	23

2.12 V500R007C71SPH105	25

2.13 V500R007C71SPH102	29

A Third-Party Software Vulnerabilities Fixes	30

B Acronyms and Abbreviations	31

# Patch Overview

1.1  Basic Information

1.2  Impact of Patch Installation

1.3  Version Compatibility Requirements

## Basic Information

Basic information about OceanStor V5 series storage systems V500R007C71SPH152.

| Patch Name | OceanStor V5 series storage systems V500R007C71SPH152 |
| --- | --- |
| Product Version | OceanStor V5 series storage systems V500R007C71SPC100 |
| Release Date | 2024-05-15 |
| Compatibility with Other Patches | patch is compatible with the patch as follow: OceanStor V5 series storage systems V500R007C71SPH150 OceanStor V5 series storage systems V500R007C71SPH137 OceanStor V5 series storage systems V500R007C71SPH136 OceanStor V5 series storage systems V500R007C71SPH132 OceanStor V5 series storage systems V500R007C71SPH130 OceanStor V5 series storage systems V500R007C71SPH120 OceanStor V5 series storage systems V500R007C71SPH115 OceanStor V5 series storage systems V500R007C71SPH112 OceanStor V5 series storage systems V500R007C71SPH111 OceanStor V5 series storage systems V500R007C71SPH107 OceanStor V5 series storage systems V500R007C71SPH105 OceanStor V5 series storage systems V500R007C71SPH102 |

## Impact of Patch Installation

### Impact on the System During Installation

Impact on Services

None.

Impact on Network Communication

None.

Impact on Command-Line Interface (CLI)

None.

### Impact on the System After Installation

None.

## Version Compatibility Requirements

The mapping relationships are consistent with those of OceanStor V5 Series Storage Systems V500R007C71SPC100.

# Resolved Issues

2.1  V500R007C71SPH152

2.2  V500R007C71SPH150

2.3  V500R007C71SPH137

2.4  V500R007C71SPH136

2.5  V500R007C71SPH132

2.6  V500R007C71SPH130

2.7  V500R007C71SPH120

2.8  V500R007C71SPH115

2.9  V500R007C71SPH112

2.10  V500R007C71SPH111

2.11  V500R007C71SPH107

2.12  V500R007C71SPH105

2.13  V500R007C71SPH102

## V500R007C71SPH152

| Trouble Ticket Number | DTS2024041206289 |
| --- | --- |
| Description | The controller enclosure is powered off unexpectedly. |
| Condition | The capacitor of the power-off switch circuit on the controller enclosure fails (this occurs with an extremely low probability), pulling down the power-off control level. |
| Impact | The controller enclosure is powered off unexpectedly, and services are interrupted. |
| Module | DMI |
| Severity | Major |
| Root Cause | According to the current power-off policy, the controller enclosure will be powered off when the capacitor fails. |
| Solution | Disable the function of powering off the device by pressing the power button by default. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024032811665 |
| --- | --- |
| Description | Importing the DeviceManager certificate using the CLI fails. |
| Condition | Use the CLI to replace the DeviceManager certificate. |
| Impact | The DeviceManager certificate cannot be replaced. |
| Module | CFG |
| Severity | Major |
| Root Cause | The original command cannot be used to replace the DeviceManager certificates of all nodes. |
| Solution | Add a minisystem command for DeviceManager certificate replacement. |
| Solution Impact | None |

## V500R007C71SPH150

| Trouble Ticket Number | DTS2024012511904 |
| --- | --- |
| Description | The database is not running properly. As a result, an error is reported during NAS share access, and DeviceManager access times out. |
| Condition | There is a low probability that this issue occurs when a large number of concurrent CCDB access requests are sent. |
| Impact | The CCDB cannot provide services after database restoration fails. As a result, the CCDB fails to be accessed during service access and an error is reported. |
| Module | CCDB |
| Severity | Major |
| Root Cause | There is a low probability that concurrent access causes the database status to be abnormal and cannot provide services. As a result, an error is reported during NAS service access. |
| Solution | Modify the logic for processing abnormal DB status, and enhance the self-recovery capability of the CCDB. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2023062702953 |
| --- | --- |
| Description | It takes a long time to run commands to query performance data in batches through the REST interface. |
| Condition | The /performance_data interface is used to query performance data in batches. |
| Impact | Querying performance statistics through the REST interface is slow, and the average interface response time exceeds 5 seconds. |
| Module | OMM |
| Severity | Minor |
| Root Cause | The polling requests wait for a long time. |
| Solution | Optimize the time required for batch query of performance data through REST interfaces. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024012309517 |
| --- | --- |
| Description | The storage CPU usage and disk IOPS surge in a short period of time. |
| Condition | Services are running properly on 4K disks, and slow disks exist. |
| Impact | The CPU usage surges unexpectedly within 1 minute. |
| Module | Disk |
| Severity | Minor |
| Root Cause | The processing logic of 4K disks is improper when I/Os are tested in the background. As a result, the CPU usage and IOPS are high. |
| Solution | Optimize the processing logic. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024012405091 |
| --- | --- |
| Description | A maintenance administrator cannot use SmartKit to perform inspection and routine O&M. |
| Condition | The maintenance administrator logs in to SmartKit. |
| Impact | The maintenance administrator has no permission to run some check commands. As a result, some inspection tasks fail. |
| Module | OMM |
| Severity | Minor |
| Root Cause | The execution of some commands requires administrator permissions. |
| Solution | Modify the permission requirements of commands. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024011904725 |
| --- | --- |
| Description | The syslog server does not report storage alarms. |
| Condition | Syslog notification is configured and an incompatible syslog server is connected to. |
| Impact | The alarm syslog function is incompatible with some syslog servers. As a result, the alarms cannot be reported. |
| Module | OMM |
| Severity | Minor |
| Root Cause | The alarm syslog is incompatible with some syslog servers. |
| Solution | Optimize the interworking with syslog servers. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2024021500349 |
| --- | --- |
| Description | Fault detection policies for silent corruptions of the system disk are optimized. |
| Condition | The system disk experiences verification errors. |
| Impact | No internal recovery mechanism is available when a system disk of a controller is faulty. |
| Module | FDSA |
| Severity | Major |
| Root Cause | Silent corruptions occur on some partitions and files of the system dis. However, only an alarm is generated, and no recovery mechanism is available. |
| Solution | Add detection and recovery measures for verification errors in key partitions of the system disk. |
| Solution Impact | None |

| Trouble Ticket Number | DTS2023121815044 |
| --- | --- |
| Description | A memory fault alarm is generated on the storage device. |
| Condition | A correctable error occurs on the memory. |
| Impact | The storage system reports a correctable memory error alarm. |
| Module | Pangu |
| Severity | Minor |
| Root Cause | The number of correctable errors in the memory exceeds the threshold. As a result, an alarm is reported. |
| Solution | Optimize the memory fault prediction algorithm to prevent false alarms caused by correctable memory errors. |
| Solution Impact | None |

## V500R007C71SPH137

| Description | The performance of the foreground service on the host decreases due to a large number of unmap services. |
| --- | --- |
| Severity | Major |
| Root Cause | The unmap services occupy too many resources. |
| Solution | Optimize the unmap processing mechanism to prevent unmap services from preempting system resources and deteriorating host service performance. |
| Solution Impact | None |

## V500R007C71SPH136

| Description | When DME is connected to a storage device, a performance threshold alarm is falsely reported. |
| --- | --- |
| Severity | Minor |
| Root Cause | The algorithm used by DME to obtain storage performance statistics is defective. |
| Solution | Optimize the algorithm used by DME to obtain storage performance statistics. |
| Solution Impact | None |

| Description | The host performance statistics cannot be queried through the RESTful API for batch query. |
| --- | --- |
| Severity | Minor |
| Root Cause | Host statistics are not included in external performance statistics. |
| Solution | Add the function of querying host performance statistics. |
| Solution Impact | None |

| Description | The iSCSI replication link resource object with the same name cannot be viewed through SystemReporter. |
| --- | --- |
| Severity | Minor |
| Root Cause | The name of the iSCSI replication link resource object already exists and is deduplicated. |
| Solution | Optimize the name of the iSCSI replication link resource object. |
| Solution Impact | None |

| Description | When the network fluctuates, SystemReporter reports a device offline alarm. |
| --- | --- |
| Severity | Minor |
| Root Cause | The device online detection interface fails to be invoked due to network packet loss, and no retry is performed. |
| Solution | The retry mechanism is added to the scenario of invoking of the device online detection interface. |
| Solution Impact | None |

| Description | The Tomcat version of SystemReporter is outdated. |
| --- | --- |
| Severity | Minor |
| Root Cause | The Tomcat version number is fixed to 1 in the SystemReporter. |
| Solution | Displays the actual Tomcat version. |
| Solution Impact | None |

| Description | When an NTP server configured with a flow control policy is connected, an alarm indicating that the NTP server is abnormal may be reported. |
| --- | --- |
| Severity | Major |
| Root Cause | The storage device's querying of the time difference between it and the NTP server exceeds the traffic control limit. As a result, access to the NTP server may fail, and an alarm indicating that the NTP server is abnormal is reported. |
| Solution | Optimize the frequency of accessing the NTP server. |
| Solution Impact | None |

| Description | When users concurrently log in to the system, there is a low probability that internal files become abnormal. |
| --- | --- |
| Severity | Minor |
| Root Cause | When multiple domain users concurrently log in to the storage system, there is an extremely low probability that internal files fail to be written. |
| Solution | Optimize the processing method in concurrent login scenarios. |
| Solution Impact | None |

| Description | After write protection is enabled for a single file system, the entire system switches to the write-through mode and the performance deteriorates. |
| --- | --- |
| Severity | Major |
| Root Cause | The number of failure pages exceeds the quota, triggering global write through. |
| Solution | Optimize the write through logic. |
| Solution Impact | None |

| Description | When the 3-way backup mode is used in NDMP scenarios, the backup fails. |
| --- | --- |
| Severity | Major |
| Root Cause | When the client request is interrupted abnormally, the status of the port provided for backup is not reset. |
| Solution | Optimize the port status change process. |
| Solution Impact | None |

| Description | The function of reporting an alarm when a SAS card is repaired must be added. |
| --- | --- |
| Severity | Minor |
| Root Cause | The function of reporting an alarm when a SAS card is repaired must be added. |
| Solution | Add the function of reporting an alarm when a SAS card is repaired. |
| Solution Impact | None |

| Description | The controller restarts after the client delivers the Setinfo command word to pipe files. |
| --- | --- |
| Severity | Major |
| Root Cause | When the client delivers the Setinfo command word to pipe files, the storage device does not intercept the command word. As a result, an exception occurs and the controller is restarted. |
| Solution | Change the process of processing the Setinfo command word for pipe files. |
| Solution Impact | None |

| Description | When the SmartQoS policy of a single file system takes effect, the performance of other services in the storage system may be affected. |
| --- | --- |
| Severity | Major |
| Root Cause | After flow control is performed for a single file system, if the number of connections is too large, the concurrency resources of a single controller are occupied. As a result, the performance of other file systems is affected. |
| Solution | Optimize the performance flow control logic. After the patch upgrade, a new policy can be configured using a switch. |
| Solution Impact | None |

| Description | There is a possibility that the stripe verification fails when RAID 6 (17 member disks) is configured. |
| --- | --- |
| Severity | Major |
| Root Cause | When RAID 6 (17 member disks) is configured, reading the parity bit times out in read-modify-write mode and the process is retried. There is a possibility that the stripe lock range is incorrect. As a result, a verification failure is falsely reported. |
| Solution | Optimize the stripe consistency verification process. |
| Solution Impact | None |

## V500R007C71SPH132

| Description | Microsoft CVE-2022-38023 forcibly enables the Netlogon RPC Sealing capability, which causes a failure for the storage system in accessing the Windows AD domain server in NTLM authentication mode. |
| --- | --- |
| Severity | Major |
| Root Cause | NTLM authentication does not support the Netlogon RPC Sealing capability. As a result, NTLM authentication of the AD domain fails. |
| Solution | Enable NTLM authentication to support the Netlogon RPC Sealing capability. |
| Solution Impact | None |

## V500R007C71SPH130

| Description | The setting of SSD timeout threshold requires an additional command. |
| --- | --- |
| Severity | Major |
| Root Cause | The setting of SSD timeout threshold can reduce the impact on upper-layer services in case of exceptions. |
| Solution | Optimize the setting of SSD timeout threshold. |
| Solution Impact | None |

| Description | When a faulty disk in a disk domain is replaced, there is a low probability that the controller resets. |
| --- | --- |
| Severity | Major |
| Root Cause | The memory resources corresponding to the faulty disk are concurrently accessed, and resource exclusion is abnormal. |
| Solution | Optimize memory resource access. |
| Solution Impact | None |

| Description | After asynchronous remote replication for SAN is configured and the secondary end is set to readable and writable, services fail to be started after synchronization is complete. |
| --- | --- |
| Severity | Major |
| Root Cause | After asynchronous remote replication for SAN is configured and the secondary end is set to readable and writable, there is a low probability that the synchronization progress is incorrectly updated. As a result, an error is reported when services are started. |
| Solution | Optimize the logic for processing progress updates. |
| Solution Impact | None |

| Description | When VAAI services are executed in some special configurations, there is a possibility that the storage controller resets. |
| --- | --- |
| Severity | Major |
| Root Cause | The VAAI read service logic does not consider exceptions under special configurations. As a result, the controller resets. |
| Solution | Optimize the VAAI read service processing logic in special configurations. |
| Solution Impact | None |

| Description | After 3DC is configured for NAS services by using remote replication, there is a possibility that synchronization fails. |
| --- | --- |
| Severity | Major |
| Root Cause | After 3DC is configured for NAS services by using remote replication, snapshot resource leakage occurs during synchronization, causing the synchronization failure. |
| Solution | Modify the snapshot processing logic. |
| Solution Impact | None |

| Description | In NAS HyperMetro scenarios, some VMware versions fail to check the client ID of the NFSv4.1 mount points. As a result, service access fails. |
| --- | --- |
| Severity | Major |
| Root Cause | The client ID composition of NFSv4.1 shares is inconsistent between the HyperMetro storage arrays. As a result, the VMware check fails. |
| Solution | Optimize the method for numbering client IDs. |
| Solution Impact | None |

| Description | The client fails to create a file. |
| --- | --- |
| Severity | Minor |
| Root Cause | The packet delivered by the client software does not comply with the protocol. As a result, the Blob Offset verification fails and the file fails to be created. |
| Solution | Optimize the tolerance of Blob Offset verification for packets that do not comply with the protocol. |
| Solution Impact | None |

| Description | When network management software (NMS) queries storage information using SNMP, there is a low probability that the query times out. |
| --- | --- |
| Severity | Minor |
| Root Cause | When a large number of concurrent queries are performed on the NMS side, there is a possibility that SNMP routine test messages time out due to a long queuing time. As a result, the SNMP service self-healing occurs. |
| Solution | Optimize the conditions for self-healing the SNMP service in a routine test task. |
| Solution Impact | None |

| Description | The audit log does not record the full share paths. |
| --- | --- |
| Severity | Minor |
| Root Cause | The maximum length of file paths is limited to 128 bytes. |
| Solution | Optimize the limitation on the file path length. |
| Solution Impact | None |

| Description | OceanStor storage systems do not provide the firewall IP address trustlist function for management ports. |
| --- | --- |
| Severity | Minor |
| Root Cause | OceanStor storage systems do not provide the firewall IP address trustlist function for management ports. |
| Solution | The minisystem provides a firewall IP address whitelist for management ports. Users can add, delete, and query the IP address whitelist of the management ports. |
| Solution Impact | None |

| Description | The time in the emails sent by the alarm notification function to the email client is incorrect. |
| --- | --- |
| Severity | Minor |
| Root Cause | Some email systems have different compatibility with the time format in alarm emails. As a result, the email time is incorrectly displayed. |
| Solution | Adjust the format of the time field in the email title. |
| Solution Impact | None |

| Description | There is a possibility that a power module on a storage controller or smart disk enclosure becomes faulty. If two power modules become faulty at the same time, the controller enclosure or smart disk enclosure goes offline. |
| --- | --- |
| Severity | Major |
| Root Cause | Conductive foreign objects in the environment may enter the power modules, causing power module faults. In addition, the time-current curve of the power module fuse partially overlaps with that of the PDU circuit breaker, which may cause the PDU circuit breaker to trip. If both power modules are faulty due to conductive foreign objects, services are interrupted. |
| Solution | Enhance the withstand voltage of the power modules to lower the failure rate in environments with conductive foreign objects. |
| Solution Impact | None |

| Description | No alarm is reported if the CCDB backup is not generated. |
| --- | --- |
| Severity | Major |
| Root Cause | The storage system does not report alarms for CCDB backup failures. |
| Solution | Add alarms for CCDB backup failures. |
| Solution Impact | None |

| Description | A CCDB exception affects NAS service access. |
| --- | --- |
| Severity | Major |
| Root Cause | The storage system does not have a self-healing mechanism in the event of CCDB exceptions. |
| Solution | Add a self-healing mechanism to proactively rectify CCDB exceptions. |
| Solution Impact | None |

## V500R007C71SPH120

| Description | A disk responds slowly and is not removed in time, resulting in slow response to host I/Os. |
| --- | --- |
| Severity | Major |
| Root Cause | The disk does not meet the conditions for being identified as a slow disk. The algorithm for determining slow disks does not remove the disk. |
| Solution | Optimize the algorithm for determining slow disks and allow parameter customization for determining slow disks. |
| Solution Impact | None |

| Description | When only one controller is running on the storage system, most hosts do not report alarms about the lack of link redundancy. |
| --- | --- |
| Severity | Minor |
| Root Cause | The multipathing software sends host link redundancy information to the protocol. The parameters for the protocol to send alarms are incorrect. As a result, the alarm module considers that the alarms reported by different hosts are from the same host and does not send alarms repeatedly. |
| Solution | Delete the second alarm parameter transferred to the alarm interface when the interface is invoked. In this case, hostId is used as the location parameter to distinguish different alarm objects. |
| Solution Impact | None |

| Description | A large number of asynchronous remote replications are configured. After the secondary end is set to be readable and writable and synchronization is started, a message is displayed indicating that the system is busy. |
| --- | --- |
| Severity | Minor |
| Root Cause | A large number of asynchronous remote replications are configured. After the secondary end is set to be readable and writable and synchronization is started, there is a low probability that resource allocation fails and a message is displayed indicating that the system is busy. |
| Solution | Optimize the logic for processing resource application failures. |
| Solution Impact | None |

| Description | After a disk domain with four disks is created, an alarm is falsely reported, which indicates that the number of member disks in the disk domain does not meet reliability requirements. |
| --- | --- |
| Severity | Minor |
| Root Cause | The alarm reporting logic is incorrectly processed in this situation. As a result, an alarm is incorrectly generated. |
| Solution | Optimize the alarm indicating that the number of member disks in the disk domain does not meet reliability requirements. |
| Solution Impact | None |

| Description | When ACL permissions are configured for files and a large number of files are frequently deleted, the space reclamation speed of the file system decreases. As a result, the free space of the file system cannot be released in time. |
| --- | --- |
| Severity | Minor |
| Root Cause | The speed of reclaiming ACL information is limited by flow control. As a result, space is not reclaimed in time. |
| Solution | Provide a command to increase the speed of reclaiming ACL information. |
| Solution Impact | None |

| Description | When the power input is intermittently disconnected frequently, the system does not report alarms indicating there is no power input. |
| --- | --- |
| Severity | Minor |
| Root Cause | The power input is intermittently disconnected and then recovered immediately before the alarm is reported by the system. |
| Solution | Optimize the alarm policy for intermittent power input disconnection. |
| Solution Impact | None |

## V500R007C71SPH115

| Description | There is a low probability that the disk fails to be repaired in the event of a soft error. |
| --- | --- |
| Severity | Major |
| Root Cause | The algorithm for repairing disk soft errors is defective. As a result, the disk fails to be repaired after a soft error occurs. |
| Solution | Optimize the algorithm for repairing disk soft errors. |
| Solution Impact | None |

| Description | After ports are bonded across interface modules, HyperMetro services may be suspended for a long time on the interface modules where bond member ports reside. |
| --- | --- |
| Severity | Major |
| Root Cause | When ports are bonded across interface modules, if the interface modules where the bond member ports reside are removed and installed again, the MAC address of the bond port changes, and ARP packets are sent to the host. |
| Solution | Prohibit inactive LIFs from sending ARP packets. |
| Solution Impact | None |

| Description | High-end bays and controllers use different policies to determine whether to turn on the red indicator for masked major alarms. |
| --- | --- |
| Severity | Major |
| Root Cause | The high-end bays take masked alarms into consideration when determining whether to turn on the alarm indicator. |
| Solution | If major or higher-level alarms are masked and there are no other alarms in the system, the alarm indicator of the bay should be off. |
| Solution Impact | None |

| Description | The statistics on the available capacity of the file system are inaccurate. |
| --- | --- |
| Severity | Major |
| Root Cause | The method for calculating the metadata space of the file system is incorrect. As a result, the available capacity of the file system is incorrectly calculated. |
| Solution | Modify the method for calculating the metadata space of the file system. |
| Solution Impact | None |

| Description | There is a low probability that controller self-healing occurs when the NFSv4 service is running. |
| --- | --- |
| Severity | Major |
| Root Cause | When the NFSv4 service is running, there is a low probability that invalid memory is accessed during asynchronous processing. |
| Solution | Optimize the memory processing in the NFSv4 service process. |
| Solution Impact | None |

| Description | When a file system is cloned, there is a low probability that the system responds slowly when the client accesses NFS shares. |
| --- | --- |
| Severity | Major |
| Root Cause | Cloning a file system triggers the update of the NFS permission cache. The domain controller query takes a long time, which affects the latency of the NFS share. |
| Solution | Optimize the mechanism for updating the NFS permission cache. |
| Solution Impact | None |

| Description | When XCOPY is used to access the CIFS shared directory, there is a low probability that the access fails. |
| --- | --- |
| Severity | Major |
| Root Cause | XCOPY delivers extra command words to modify the share properties. As a result, the access is abnormal. |
| Solution | Correctly process the command that modifies the properties of a shared directory. |
| Solution Impact | None |

## V500R007C71SPH112

| Description | The host uses the NFSv4 protocol to lock files. After the controller where the logical IP address resides is restarted, there is a possibility that the client fails to re-apply for the lock from the new controller. |
| --- | --- |
| Severity | Major |
| Root Cause | When the client re-applies for the lock from the storage system, there is a time sequence error between the client re-application time and the storage system silent time. If the time sequence is out of order, the lock re-application may fail. |
| Solution | Correctly process the lock re-application time sequence when the client re-applies for the lock from the storage system. |
| Solution Impact | None |

## V500R007C71SPH111

| Description | There is a possibility that false alarms are generated for controllers and expansion modules after the system runs for a long time. |
| --- | --- |
| Severity | Minor |
| Root Cause | After the system runs for a long time, the alarm check policy is incorrectly determined. As a result, a false alarm is generated. |
| Solution | Optimize the alarm check policy after long-term running. |
| Solution Impact | None |

| Description | If a power outage occurs during the process of an expected power-off on a controller, there is a low probability that the system fails to be started. |
| --- | --- |
| Severity | Minor |
| Root Cause | If a power outage occurs during the process of an expected power-off on a controller, there is a low probability that the system fails to be started due to a process conflict. |
| Solution | Optimize the concurrent processing of controller power-off and power outage. |
| Solution Impact | None |

| Description | The power module version cannot be displayed after the spare power module is powered on. |
| --- | --- |
| Severity | Minor |
| Root Cause | After spare part replacement, the firmware version synchronization and query processes are initiated concurrently. As a result, there is a possibility that the queried power module version is invalid. |
| Solution | Optimize the policy for querying the power module version of a disk enclosure. |
| Solution Impact | None |

| Description | After the recovery mode of a heterogeneous iSCSI link is changed to manual and the primary controller connected to the iSCSI initiator is restarted, the heterogeneous link cannot be reconnected or modified. |
| --- | --- |
| Severity | Minor |
| Root Cause | When the recovery mode of the iSCSI target is manual, the BDM interface is not invoked to restore data to the memory object during the power-on of storage arrays. As a result, the corresponding link information cannot be obtained during modification and reconnection. |
| Solution | During the power-on process, if the recovery mode of the iSCSI target is manual, the BDM interface should also be invoked to restore the corresponding link information to the memory object. |
| Solution Impact | None |

| Description | The internal communication process of DeviceManager restarts unexpectedly. As a result, the external NMS is disconnected. |
| --- | --- |
| Severity | Major |
| Root Cause | An exception occurs when the DeviceManager communication process works with the request delivered by the external NMS for querying real-time performance data. As a result, the memory usage of the DeviceManager communication process is too high, causing the restart of the process. |
| Solution | Optimize the communication process between DeviceManager and the external NMS. |
| Solution Impact | None |

| Description | No data is returned when eSight is used to query the performance data of disks in the IP storage enclosures. |
| --- | --- |
| Severity | Major |
| Root Cause | Performance data of disks in IP enclosures cannot be obtained. |
| Solution | Add the function of obtaining performance data of disks in IP enclosures. |
| Solution Impact | None |

| Description | Fields in the certificate export request can be customized. |
| --- | --- |
| Severity | Minor |
| Root Cause | Fields in the certificate export request can be customized. |
| Solution | Fields in the certificate export request can be customized. |
| Solution Impact | None |

| Description | Add the function of displaying the total capacity of disks provided by different vendors on DeviceManager. |
| --- | --- |
| Severity | Info |
| Root Cause | Add the function of displaying the total capacity of disks provided by different vendors on DeviceManager. |
| Solution | Add the function of displaying the total capacity of disks provided by different vendors on DeviceManager. |
| Solution Impact | None |

| Description | The severity is incorrect for the disk fault alarm reported after the disk self-check fails. |
| --- | --- |
| Severity | Info |
| Root Cause | If a disk is faulty, self-check and recovery are performed. If the self-check and recovery fail, the disk cannot be accessed. The alarm severity reported in the exception handling process is incorrect. |
| Solution | Optimize the disk alarm logic during disk diagnosis. |
| Solution Impact | None |

| Description | The splitting speed of a file system clone is inconsistent with that described on DeviceManager. |
| --- | --- |
| Severity | Minor |
| Root Cause | When the service load is heavy, the splitting speed of the file system clone is controlled. As a result, it is different from that displayed on the GUI. |
| Solution | Adjust the duration of file system clone splitting to improve the splitting speed. Adjust the description on DeviceManager to match the splitting speed. |
| Solution Impact | None |

| Description | The number of CIFS and NFS connections should be increased. |
| --- | --- |
| Severity | Minor |
| Root Cause | The number of CIFS and NFS connections should be increased to 32,000. |
| Solution | For OceanStor 5500 V5, increase the number of connections to 32,000 per controller. |
| Solution Impact | None |

| Description | In the concurrent directory listing scenario, there is a low probability that resource application fails. Memory resources are not cleared in the exception handling process. |
| --- | --- |
| Severity | Minor |
| Root Cause | In the concurrent directory listing scenario, there is a low probability that resource application fails. Memory resources are not cleared in the exception handling process. |
| Solution | Clear the resources in the exception handling process. |
| Solution Impact | None |

## V500R007C71SPH107

| Description | When a large number of files are deleted from a host, there is a low probability that the HyperMetro pair is disconnected. |
| --- | --- |
| Severity | Minor |
| Root Cause | A large number of background deletion I/Os cause high latency in disk flushing at the remote end. As a result, writing to the remote end times out, and then the HyperMetro pair is disconnected. |
| Solution | Optimize the logic for processing background I/Os in the HyperMetro scenario. |
| Solution Impact | None |

| Description | The failure of driver resource application causes a low probability of intermittent link disconnection. |
| --- | --- |
| Severity | Minor |
| Root Cause | The failure of driver resource application causes a low probability of intermittent link disconnection. |
| Solution | Optimize the process for handling exceptions in driver resource application. |
| Solution Impact | None |

| Description | There is a low probability that a hardware fault in a SAS chip causes high I/O latency. |
| --- | --- |
| Severity | Minor |
| Root Cause | The hardware exception handling implemented by the driver is defective. |
| Solution | Optimize the hardware exception handling implemented by the driver. |
| Solution Impact | None |

| Description | When the replication distance is long, the maximum IP network bandwidth is not reached. |
| --- | --- |
| Severity | Minor |
| Root Cause | When the replication distance is long, the link latency is high. The transmission efficiency decreases due to improper buffer settings. |
| Solution | Optimize the buffer settings for the scenario of long replication distance. |
| Solution Impact | None |

| Description | The host sends a large number of abnormal TMF commands. As a result, too many driver resources are occupied. |
| --- | --- |
| Severity | Minor |
| Root Cause | The host sends a large number of abnormal TMF commands. As a result, too many driver resources are occupied. |
| Solution | Optimize the driver resource usage for abnormal TMF commands. |
| Solution Impact | None |

| Description | The SNMP process of the storage system is reset. |
| --- | --- |
| Severity | Minor |
| Root Cause | The open-source software SNMP++ used by SNMP has a memory leak problem. As a result, the SNMP agent process consumes more and more memory. When the memory usage reaches the upper limit (100 MB) set for the storage system, the storage system triggers the SNMP process reset to rectify the fault. |
| Solution | Update the open-source software SNMP++ to resolve the memory leak problem. |
| Solution Impact | None |

| Description | When the number of files opened using SMB 2.0 reaches the upper limit, more files fail to be opened. |
| --- | --- |
| Severity | Major |
| Root Cause | When the number of files opened using SMB 2.0 reaches the upper limit, new services fail to apply for memory. |
| Solution | Raise the upper limit of the number of files that can be opened using SMB 2.0. |
| Solution Impact | None |

| Description | There is a low probability that CIFS access fails in the event of an exception when the storage system is added to an AD domain for the first time. |
| --- | --- |
| Severity | Major |
| Root Cause | When the storage system is added to the AD domain for the first time, the link is intermittently disconnected when the storage system communicates with the domain controller. An incorrect SID is transferred, causing CIFS access failure. |
| Solution | Optimize the exception handling process. |
| Solution Impact | None |

## V500R007C71SPH105

| Description | If the communication between a controller and an expansion module becomes abnormal multiple times, there is a low probability that the expansion module resets. |
| --- | --- |
| Severity | Minor |
| Root Cause | If the communication is repeatedly abnormal for a long time, there is a low probability that a mechanism is triggered to reset the expansion module for repair purposes. |
| Solution | Optimize the mechanism for handling abnormal communication. |
| Solution Impact | None |

| Description | Under heavy workloads, there is a low probability that distributed locking causes a controller to reset, resulting in service interruption. |
| --- | --- |
| Severity | Major |
| Root Cause | Under heavy workloads, if a failed node is recovered, other nodes will pack and return the lock resources previously owned by that node. However, if a large amount of lock resources need to be packed, there is a low probability of space calculation errors, causing the subsequent process to access invalid addresses. |
| Solution | Optimize the policy for distributed locking to properly handle node failure and recovery under heavy workloads. |
| Solution Impact | None |

| Description | A disk in a high-density enclosure triggers a high-temperature alarm. |
| --- | --- |
| Severity | Minor |
| Root Cause | Under long-term heavy workloads, the fan speed is not adjusted in a timely manner. As a result, the high temperature alarm is triggered. |
| Solution | Optimize the fan speed adjustment policy for high-density enclosures. |
| Solution Impact | None |

| Description | In the 3DC scenario, synchronization is started after sites are disconnected. Then, there is a low probability that a controller resets. |
| --- | --- |
| Severity | Major |
| Root Cause | After sites A and B are disconnected and take over services, data is synchronized between sites B and C first, and then between sites A, B, and C. In this case, there is a low probability that site C resets due to a time sequence error during the synchronization. |
| Solution | Optimize the time sequence processing algorithm. |
| Solution Impact | None |

| Description | Imbalanced host I/O workloads cause CPU partitions to be overloaded. |
| --- | --- |
| Severity | Major |
| Root Cause | In a specific I/O model, the load balancing algorithm fails to fully balance loads between CPU partitions. |
| Solution | Optimize the load balancing algorithm. |
| Solution Impact | None |

| Description | Querying information about disk groups in a disk domain fails. |
| --- | --- |
| Severity | Major |
| Root Cause | When a user attempts to query information about disk groups in a large-capacity disk domain, there is a low probability that the message communication is abnormal and the query status is not updated. As a result, the subsequent query process fails. |
| Solution | Optimize the message communication mechanism. |
| Solution Impact | None |

| Description | When a large number of Fibre Channel links are configured, some links are displayed abnormally. |
| --- | --- |
| Severity | Minor |
| Root Cause | When a large number of Fibre Channel links are configured, some links are displayed abnormally. |
| Solution | Solve the link display problem. |
| Solution Impact | None |

| Description | In a HyperMetro pair, the security mode of the secondary file system is different from that of the primary file system. |
| --- | --- |
| Severity | Minor |
| Root Cause | In the HyperMetro incremental synchronization scenario, there is a low probability that the security mode of the secondary file system is different from that of the primary file system. |
| Solution | Synchronize the correct security mode information to the secondary file system. |
| Solution Impact | None |

| Description | If a file fails to be deleted, there is a low probability that a controller resets. |
| --- | --- |
| Severity | Minor |
| Root Cause | Under heavy workloads, the process for handling file deletion timeout is defective. As a result, resources are not released in a timely manner, triggering a control reset. |
| Solution | Optimize the process for handling file deletion timeout. |
| Solution Impact | None |

| Description | The password of the super administrator can contain a minimum of eight characters. |
| --- | --- |
| Severity | Minor |
| Root Cause | The password of the super administrator can contain a minimum of eight characters. |
| Solution | The password of the super administrator can contain a minimum of eight characters. |
| Solution Impact | None |

| Description | The BMC software's flash P/E mechanism is optimized. |
| --- | --- |
| Severity | Major |
| Root Cause | The daily amount of data written by the BMC software to flash is relatively large, affecting the flash lifespan. |
| Solution | Optimize the BMC software's flash P/E mechanism to prolong the flash lifespan. |
| Solution Impact | None |

| Description | The indicator on a management module turns off unexpectedly due to an incorrect policy. |
| --- | --- |
| Severity | Minor |
| Root Cause | The indicator on a management module turns off unexpectedly due to an incorrect policy. |
| Solution | Optimize the policy for turning on and off the indicator on a management module. |
| Solution Impact | None |

| Description | A disk is incorrectly repaired due to a hardware fault. |
| --- | --- |
| Severity | Minor |
| Root Cause | A disk is incorrectly repaired due to a hardware fault. |
| Solution | Optimize the internal disk repair mechanism. |
| Solution Impact | None |

| Description | There is an extremely low probability that I/Os fail when the replication link network is congested. |
| --- | --- |
| Severity | Major |
| Root Cause | The link reset policy oriented to network congestion scenarios is abnormal, causing an extremely low probability of I/O failure. |
| Solution | Optimize the link reset policy oriented to network congestion scenarios. |
| Solution Impact | None |

## V500R007C71SPH102

This section describes the issues that are resolved in V500R007C71SPH102.

| Description | Robocopy failed to migrate ADS with the directory. |
| --- | --- |
| Severity | Minor |
| Root Cause | During ADS flow migration, abnormal branches are incorrectly verified. As a result, the migration fails. |
| Solution | Remove the incorrectly check for directory ADS flows. |
| Solution Impact | None |

Third-Party Software Vulnerabilities Fixes

Vulnerabilities information is available using CVE IDs on the National Vulnerability Database (NVD) website: http://web.nvd.nist.gov/view/vuln/search.

| Software Name | Version | CVE ID | Vulnerability Description | Solution |
| --- | --- | --- | --- | --- |
| - | - | - | - | - |

Acronyms and Abbreviations

None.