|  | HUAWEI VP9660, VP9650, VP9630 V600R019C10SPCd00 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Release Notes |  |  |  |
|  | Issue | 01 |  |  |
|  | Date | 2021-03-27 |  |  |
|  | HUAWEI TECHNOLOGIES CO., LTD. |  |  |  |

Copyright © Huawei Technologies Co., Ltd. 2021. All rights reserved. No part of this document may be reproduced or transmitted in any form or by any means without prior written consent of Huawei Technologies Co., Ltd. Trademarks and Permissions and other Huawei trademarks are trademarks of Huawei Technologies Co., Ltd. All other trademarks and trade names mentioned in this document are the property of their respective holders. Notice The purchased products, services and features are stipulated by the contract made between Huawei and the customer. All or part of the products, services and features described in this document may not be within the purchase scope or the usage scope. Unless otherwise specified in the contract, all statements, information, and recommendations in this document are provided "AS IS" without warranties, guarantees or representations of any kind, either express or implied. The information in this document is subject to change without notice. Every effort has been made in the preparation of this document to ensure accuracy of the contents, but all statements, information, and recommendations in this document do not constitute a warranty of any kind, express or implied.

| Huawei Technologies Co., Ltd. |  |
| --- | --- |
| Address: | Huawei Industrial Base Bantian, Longgang Shenzhen 518129 People's Republic of China |
| Website: | https://e.huawei.com |

Contents

1 Version Requirements	1

1.1 Product Version	1

1.2 Version Evolution	1

1.3 Software Versions	2

1.4 Hardware Versions	2

1.5 Related Product Versions	3

1.6 Software List	3

1.7 Virus Scan Result	3

2 Version Compatibility	4

2.1 Software Version Compatibility	4

2.2 Hardware Version Compatibility	4

3 Important Notes	5

3.1 Precautions	5

3.2 Restrictions	5

4 V600R019C10SPCd00	6

4.1 What's New	6

4.2 Resolved Issues	6

4.3 Version Constraints	6

4.4 Known Issues	7

5 V600R019C10SPCc00	8

5.1 Feature Description	8

5.2 Resolved Issues	8

5.3 Version Constraints	9

5.4 Known Issues	9

6 V600R019C10SPC900	10

6.1 What's New	10

6.2 Resolved Issues	10

6.3 Version Constraints	11

6.4 Known Issues	12

7 V600R019C10SPC800	13

7.1 What's New	13

7.2 Resolved Issues	13

7.3 Version Constraints	13

7.4 Known Issues	14

8 V600R019C10SPC750	15

8.1 What's New	15

8.2 Resolved Issues	15

8.3 Known Issues	16

8.4 Known Issues	16

9 V600R019C10SPC731	17

9.1 What's New	17

9.2 Resolved Issues	17

9.3 Known Issues	17

10 V600R019C10SPC730	18

10.1 What's New	18

10.2 Resolved Issues	18

10.3 Known Issues	18

11 V600R019C10SPC731	19

11.1 What's New	19

11.2 Resolved Issues	19

11.3 Known Issues	19

12 V600R019C10SPC730	20

12.1 What's New	20

12.2 Resolved Issues	20

12.3 Known Issues	20

13 V600R019C10SPC610	21

13.1 What's New	21

13.2 Resolved Issues	21

13.3 Known Issues	21

14 V600R019C10SPC600	22

14.1 What's New	22

14.2 Resolved Issues	22

14.3 Known Issues	23

15 V600R019C10SPC300	24

15.1 What's New	24

15.2 Resolved Issues	24

15.3 Known Issues	24

16 V600R019C10SPC100	25

16.1 What's New	25

16.2 Resolved Issues	25

16.3 Known Issues	25

17 V600R019C10	26

17.1 What's New	26

17.2 Resolved Issues	27

17.3 Known Issues	27

18 Upgrade Impact	28

18.1 Impact on the System During the Upgrade	28

18.1.1 Impact on Services	28

18.1.2 Impact on Network Communication	28

18.2 Impact on the System After the Upgrade	28

19 Related Documentation	29

19.1 V600R019C10 Documentation	29

19.2 Obtaining Documentation	31

19.2.1 Support-E Website	31

19.2.2 Support Website	31

19.2.3 Hard Copies	31

20 Appendix	32

20.1 Third-Party Software Vulnerability Fixes	32

20.2 Acronyms and Abbreviations	32

# Version Requirements

## Product Version

| Product | HUAWEI VP9660 |
| --- | --- |
| Product Model | HUAWEI VP9660,HUAWEI VP9650,HUAWEI VP9630 |
| Product Version | V600R019C10SPCd00 |
| Digital Signature | HUAWEI-VP9660-xxx.zip.asc |

To prevent a software package from being maliciously tampered with during transmission or storage, download the corresponding digital signature file together with the software package for integrity verification.

After the software package is downloaded, verify its PGP digital signature by referring to the OpenPGP Signature Verification Guide. If the software package fails the verification, do not use the software package, and contact Huawei technical support engineers.

Before using a software package for installation or upgrade, verify its digital signature by referring to the OpenPGP Signature Verification Guide to ensure that the software package is not tampered with. Visit either of the following websites to obtain the OpenPGP Signature Verification Guide:

Carrier customers: http://support.huawei.com/carrier/digitalSignatureAction

Enterprise users: https://support.huawei.com/enterprise/en/tool/pgp-verify-TL1000000054

## Version Evolution

← VP9660 V600R019C10SPCd00

← VP9660 V600R019C10SPCc00

← VP9660 V600R019C10SPC900

← VP9660 V600R019C10SPC800

← VP9660 V600R019C10SPC750

← VP9660 V600R019C10SPC731

← VP9660 V600R019C10SPC730

← VP9660 V600R019C10SPC720

← VP9660 V600R019C10SPC700

← VP9660 V600R019C10SPC610

← VP9660 V600R019C10SPC600

← VP9660 V600R019C10SPC300

← VP9660 V600R019C10SPC100

← VP9660 V600R019C10

## Software Versions

None

## Hardware Versions

| Board Name | Version | Remarks |
| --- | --- | --- |
| HCCA | PCB version: B PCB-BOM version: 2 Main FPGA version: 17121901 Main CPLD version: 18121301 Me version: Consumer_11.8.79.3722 | - |
| HCCB | PCB version: B PCB-BOM version: 2 Main FPGA version: 17121901 Main CPLD version: 18121301 Me version: Consumer_11.8.79.3722 | - |
| HCCC | PCB version: B PCB-BOM version: 2 Main FPGA version: 17121901 Main CPLD version: 18121301 Me version: Consumer_11.8.79.3722 | - |
| Media+ | PCB version: B PCB-BOM version: 2 Main FPGA version: 17121901 Main CPLD version: 18121301 Me version: Consumer_11.8.79.3722 | - |

## Related Product Versions

| Product | Version | Description |
| --- | --- | --- |
| SMC2.0 | SMC2.0 V600R019C10SPC400 | - |
| TE30&TE40&TE50&TE60 | TE60 V600R019C00SPC500 | - |
| TE Mobile | TE Mobile V600R006C10SPCb02 | - |
| TE Desktop | TE Desktop V600R006C10SPCb02 | - |
| TE10&TE20 | TE10 V600R019C00SPC900 | - |
| RSE6500 | RSE6500 V500R002C00SPCi00 | - |

## Software List

| Software Package | Description |
| --- | --- |
| HUAWEI-VP9660-FULL_EC.zip | MCU software upgrade package for the EC networking. The package is decompressed to extract the HUAWEI-VP9660-FULL.bin upgrade package and sha512.sum verification file. HUAWEI-VP9660-FULL.bin: MCU full upgrade package, which is used to upgrade the software version. sha512.sum: MCU upgrade package verification file. |

## Virus Scan Result

The software packages and documentation have been scanned by McAfee, Avira, Kav, Symantec, and OSCE antivirus software. No virus is found. For details, see the VP9660 V600R019C10SPCd00 Virus Scan Report.

# Version Compatibility

## Software Version Compatibility

This version supports upgrade from only the V600R006C10 or V600R019 series versions but does not support upgrade from versions earlier than V600R006C10 or V600R019.

## Hardware Version Compatibility

None.

# Important Notes

## Precautions

Before upgrading the MCU, you need to check the power supply security to prevent a power outage during upgrade.

If the IP address is correct and can be pinged but HTTP cannot be used for login during login to the built-in web of the MCU, try to use HTTPS to log in.

The built-in web server of this version supports following operating systems: Internet Explorer (9-11), Chrome (46-67), Firefox (41-60), Safari (8.0-10), and Edge (40-42).

## Restrictions

1. This software version can run only on the new MCU of the RED5.0 board but cannot be upgraded from the NP board MCU.

2. ISDN/4E1 access is not supported.

# V600R019C10SPCd00

## What's New

None

## Resolved Issues

| Trouble Ticket Number | DTS2021022303750 |
| --- | --- |
| Symptom | After a 64 kbit/s call is initiated from a Polycom endpoint, initiate a 1920 kbit/s call from the endpoint again. However, the call rate is still 64 kbit/s. As a result, a black screen is displayed. |
| Recurrence Condition | After the Polycom endpoint initiates a 64 kbit/s call, the Polycom endpoint initiates a 1920 kbit/s call again. |
| Impact | The participant fails to join the conference. |
| Severity | Black screen is displayed. |
| Root Cause | The Polycom calling party carries the H.323 ID. The system performs comparison, but the H.323 ID fails to be matched with the participant name of the 64 kbit/s call. |
| Solution | The MCU interworking and processing policy is adjusted. |
| Rectification Impact | None |

## Version Constraints

The current version does not support SIP-based participant cascading.

## Known Issues

Issue Description

| Trouble Ticket Number | DTS2019071513568 |
| --- | --- |
| Symptom | A two-level cascaded conference with universal transcoding is held. Three MCUs are involved, and all of them use the x86 architecture. In the cascaded conference, one MCU is located at the upper level and two MCUs at the lower level. Each of the two lower-level MCUs has one TE soft client. The two TE soft clients are broadcast. Their video displayed to other participants is blurry in the first two seconds after being received and then turns normal. |
| Severity | Minor |
| Workaround | None |
| Impact | After the soft client participant is switched, the image convergence time is about 2s, while the display effect is normal on the hard endpoint. |
| Progress | No workaround is provided in this version. |

# V600R019C10SPCc00

## Feature Description

None

## Resolved Issues

| Trouble Ticket Number | DTS2020101305029 |
| --- | --- |
| Symptom | In the ad hoc large-capacity scenario, if one 1080p60 participant cannot be connected to the MCU due to resource insufficiency, the participant can still be connected to the current MCU on the SMC. |
| Recurrence Condition | Connect an ad hoc conference in large-capacity scenario. |
| Impact | The participant fails to join the conference. |
| Severity | Major |
| Root Cause | The resource pre-deduction on the SMC is different from that of the MCU. When the SMC connects a participant to the MCU, the MCU has no resource. |
| Solution | Enable the SMC and MCU to use the same resource deduction policy. |
| Rectification Impact | None |

| Trouble Ticket Number | DTS2021012604138 |
| --- | --- |
| Symptom | [VC 20.1] [VP9660] [Media exchange] [Shandong video networking] The mswt module breaks down. |
| Recurrence Condition | There is a possibility that this issue occurs in scenarios with high network loss and delay. |
| Impact | The board is restarted because the mswt service breaks down. |
| Severity | Critical |
| Root Cause | The parameter validity is not verified before the MCU array is accessed. As a result, array access violation occurs. |
| Solution | The parameter validity verification is added to the code logic. |
| Rectification Impact | None |

## Version Constraints

The current version does not support SIP-based participant cascading.

## Known Issues

Issue Description

| Trouble Ticket Number | DTS2019071513568 |
| --- | --- |
| Issue Description | A two-level cascaded conference with universal transcoding is held. Three MCUs are involved, and all of them use the x86 architecture. In the cascaded conference, one MCU is located at the upper level and two MCUs at the lower level. Each of the two lower-level MCUs has one TE soft client. The two TE soft clients are broadcast. Their videos displayed to other participants are blurry in the first two seconds after being received and then become normal. |
| Severity | Minor |
| Workaround | None |
| Impact | After the soft client participant is switched, the image convergence time is about 2s, while the display effect is normal on the hard endpoint. |
| Progress | No workaround is provided in this version. |

# V600R019C10SPC900

## What's New

None

## Resolved Issues

| Trouble Ticket Number | DTS2020121407194 |
| --- | --- |
| Symptom | VP9660 V600R019C10SPC900 [Presentation] [Introduced during modification] When two participants are cascaded in hybrid mode and communicate with the MCU through H.323, if the lower-level conference shares presentation and locks the conference, the upper-level conference participant can successfully share the presentation and the two participants of the two conferences can view their own presentation. |
| Recurrence Condition | Two participants are cascaded in hybrid mode and communicate with the MCU through H.323. The lower-level conference shares presentation and locks the conference. |
| Impact | The two participants of the two conferences view their own presentation. |
| Severity | Major |
| Root Cause | The transmission of the presentation indication message is interrupted. |
| Solution | The presentation indication message is transmitted normally. |
| Rectification Impact | None |

| Trouble Ticket Number | DTS2020123105777 |
| --- | --- |
| Symptom | [Conference control] [Shandong video networking] The standby board is suspended. |
| Recurrence Condition | In a 2.0 cascaded conference, a cascaded participant joins the conference and then leaves the conference for multiple times. |
| Impact | The standby board is suspended, and an alarm is generated. Currently, services are not affected. |
| Severity | Minor |
| Root Cause | An error occurs when the standby MCU processes the access response from the active MCU. As a result, data is deleted incorrectly. |
| Solution | Modify the logic for the standby board to process access response messages. |
| Rectification Impact | None |

| Trouble Ticket Number | DTS2020122800144 |
| --- | --- |
| Symptom | [Shandong video networking] [Maintenance discovery] An exception occurs when the standby board processes data of cascaded participants. As a result, the standby board is suspended and restarts. |
| Recurrence Condition | Triggered in common service scenarios |
| Impact | The standby board is suspended, and an alarm is generated. Currently, services are not affected. |
| Severity | Minor |
| Root Cause | During the processing of cascaded conferences, an error occurred during participant information synchronization and the information about the deleted participant is processed in the standby board. |
| Solution | Add the condition for checking whether a participant exists before processing the information about the cascaded participant. |
| Rectification Impact | None |

## Version Constraints

The current version does not support SIP-based participant cascading.

## Known Issues

Issue Description

| Trouble Ticket Number | DTS2020121706834 |
| --- | --- |
| Symptom | VP9660 V600R019C10SPC900 SMC2.0 [Conference control] [Patch test] In a level-2 conference, when presentation is sent and continuous presence is broadcast, the upper-level conference preempts the presentation sharing right. When the video image is switched on a soft client in the level-2 conference, the continuous presence is displayed for about 3s. |
| Severity | Minor |
| Workaround | None |
| Impact | Preemption of the presentation sharing right in common cascading scenarios |
| Progress | The root cause has been determined, and the solution is to be determined. |

# V600R019C10SPC800

## What's New

New Features

None

## Resolved Issues

| Trouble Ticket Number | DTS2020102710834 |
| --- | --- |
| Symptom | VP9660_19.1_SMC2.0 [Maintenance feedback] In a provincial-level conference held in Shandong province, a character in the caption is incorrectly displayed. |
| Recurrence Condition | 1. Initiate a universal transcoding conference, set the Sharing mode on the Caption tab page to Other than T.140 for the endpoint, and call the endpoint to join the conference. 2. On the SMC, set the character that was incorrectly displayed to be displayed in the banner. 3. The character is incorrectly displayed in the banner on the endpoint. |
| Impact | Banner display |
| Severity | Minor |
| Root Cause | The word stock is not properly set. |
| Solution | Adjust the word stock. |
| Rectification Impact | None |

## Version Constraints

The current version does not support SIP-based participant cascading.

## Known Issues

Issue Description

| Trouble Ticket Number | DTS2020110701345 |
| --- | --- |
| Symptom | VP9660_19.1_SMC2.0 networking [Maintenance discovery] When two groups of two-level cascaded conferences are held on an MCU and the function of continuously selecting a participant to view is enabled on the SMC for the chair participant (the lower-level participant on group), manually select the lower-level participant of the other group. There is a possibility that frame freezing of the selected participant lasts for about 3 to 4 seconds. |
| Severity | Major |
| Workaround | None |
| Impact | On the SMC, switch participants consecutively. |
| Progress | The problem scope has been determined, and the root cause is being located. |

# V600R019C10SPC750

## What's New

新增特性

| No. | Feature Name | Sub-Feature Name | Feature Description |
| --- | --- | --- | --- |
| 1 | Presentation optimization for cascaded participants | - | Optimize the presentation feature design so that it is more user-friendly. |
| 2 | Conference control allowed for the chair site after the SMC that designates the site as the chair site goes offline | - | Allow the chair site to perform conference control operations during an SMC switchover. |

## Resolved Issues

| Trouble Ticket No. | DTS2020092510955 |
| --- | --- |
| Symptom | After a conference is initiated, viewing the debug log of the cpucom module on the web page causes a cpucom process restart. |
| Recurrence Condition | Initiate a conference and open the debug log of the cpucom module on the web interface. |
| Impact | The cpucom process restarts. |
| Severity | Major |
| Root Cause | Debug logs are incorrectly recorded. As a result, the process restarts. |
| Solution | Modify log printing. |
| Rectification Impact | None |

## Known Issues

None

## Known Issues

Issue Description

| Trouble Ticket Number | DTS2020110701345 |
| --- | --- |
| Symptom | VP9660_19.1_SMC2.0 networking [Maintenance discovery] When two groups of two-level cascaded conferences are held on an MCU and the function of continuously selecting a participant to view is enabled on the SMC for the chair participant (the lower-level participant on group), manually select the lower-level participant of the other group. There is a possibility that frame freezing of the selected participant lasts for about 3 to 4 seconds. |
| Severity | Major |
| Workaround | None |
| Impact | On the SMC, switch participants consecutively. |
| Progress | The problem scope has been determined. The root cause is being located and will be resolved in the next version. |

# V600R019C10SPC731

## What's New

SPC731 is the patch version of SPC730 and is used on the SMC2.0 network. Some issues are resolved based on SPC720, and no new features are added.

## Resolved Issues

| Trouble Ticket Number | DTS2020102403580 |
| --- | --- |
| Symptom | The call_media of the signaling board is suspended and the signaling board is not switched over. |
| Recurrence Condition | Kill the key processes of the signaling boards in slots 8 and 9. |
| Impact | A signaling board exception occurs. |
| Severity | Major |
| Root Cause | This issue is caused by the code conversion from int16 to unsigned int32. As a result, after the boards in slot 8 and 9 obtain the version number, an error occurs when the boards conduct code conversion. |
| Solution | The type conversion logic is modified. |
| Rectification Impact | None |

## Known Issues

None

# V600R019C10SPC730

## What's New

SPC730 is the patch version of SPC720 and is used on the SMC2.0 network. Some issues are resolved based on SPC720, and no new features are added.

## Resolved Issues

| Trouble Ticket Number | DTS2020092510955 |
| --- | --- |
| Symptom | After a conference is initiated, viewing the debug log of the cpucom module on the web page causes a cpucom process restart. |
| Recurrence Condition | Initiate a conference and open the debug log of the cpucom module on the web interface. |
| Impact | The cpucom process restarts. |
| Severity | Major |
| Root Cause | Debug logs are incorrectly recorded. As a result, the process restarts. |
| Solution | Modify log printing. |
| Rectification Impact | None |

## Known Issues

None

# V600R019C10SPC731

## What's New

SPC731 is the patch version of SPC730 and is used on the SMC2.0 network. Some issues are resolved based on SPC720, and no new features are added. l00225196,c00222590,y00195116,wwx457650

## Resolved Issues

| Trouble Ticket No. | DTS2020102403580 |
| --- | --- |
| Symptom | The call_media of the signaling board is suspended and the signaling board is not switched over. |
| Recurrence Condition | Kill the key processes of the signaling boards in slots 8 and 9. |
| Impact | A signaling board exception occurs. |
| Severity | Major |
| Root Cause | This issue is caused by the code conversion from int16 to unsigned int32. As a result, after the boards in slot 8 or 9 obtain the version number, an error occurs when the boards conduct code conversion. |
| Solution | The type conversion logic is modified. |
| Rectification Impact | None |

## Known Issues

None

# V600R019C10SPC730

## What's New

SPC731 is the patch version of SPC730 and is used on the SMC2.0 network. Some issues are resolved based on SPC720, and no new features are added. l00225196,c00222590,y00195116,wwx457650

## Resolved Issues

| Trouble Ticket Number | DTS2020041405310 |
| --- | --- |
| Symptom | Failed to add the site. |
| Condition | An H.323 terminal requests chair control rights and then adds an H.323 site. |
| Impact | Failed to add the new site. |
| Severity | Major |
| Root Cause | The MCU sends the default value to the SMC when the chair participant adds a participant. As a result, the SMC sends the default value to the MCU and adds the participant. However, the MCU does not support the H.261 capability, and the participant fails to be added. |
| Solution | The message reporting policy is modified so that the protocol field can be correctly reported to the SMC. |
| Solution Impact | None |

## Known Issues

None

# V600R019C10SPC610

## What's New

SPC610 is the patch version of SPC600. Some issues found in SPC600 are resolved in SPC610, and no new requirements are added.

## Resolved Issues

| Trouble Ticket Number | DTS2020032319708 |
| --- | --- |
| Symptom | VP9660 MCU standby board restart. |
| Condition | Adding a Participant with Two-Stage Dialing. |
| Impact | VP9660 MCU standby board restart. |
| Severity | Major |
| Root Cause | When the active control board synchronizes the passcode to the standby control board, the standby control board accesses an invalid address. |
| Solution | The standby board uses the temporary array for synchronization and does not access the address transferred from the active board. |
| Solution Impact | None |

## Known Issues

None

# V600R019C10SPC600

## What's New

SPC600 is the patch version of SPC300. Some issues found in SPC300 are resolved in SPC600, and the new feature about DH2048 is added.

New Feature

| No. | Feature | Subfeature | Description |
| --- | --- | --- | --- |
| 1 | The H323 protocol supports the DH2048 algorithm. | - | The H323 protocol supports the DH2048 algorithm. |

## Resolved Issues

| Trouble Ticket Number | DTS2019110806925 |
| --- | --- |
| Symptom | A site in a conference sends a presentation, but other sites cannot receive the presentation. |
| Condition | The Polycom endpoint calls 9000 to enter the hall to join the conference. The presentation packet loss concealment (PLC) function is not disabled completely for the presentation 16. As a result, the presentation 16 is allocated, but fails to be sent. |
| Impact | When the Polycom endpoint calls the 9000 to enter the hall, the site fails to share the presentation. |
| Severity | Minor |
| Root Cause | When the Polycom site enters the hall, the presentation receiving channel is enabled and the presentation PLC function is enabled. The presentation PLC function is not disabled when the conference is being held. |
| Solution | Process the presentation packet loss concealment correctly. |
| Solution Impact | None |

## Known Issues

None

# V600R019C10SPC300

## What's New

SPC300 is the patch version of SPC100. Some issues found in SPC100 are resolved in SPC300, and no new requirements are added.

## Resolved Issues

| Trouble Ticket Number | DTS2019080502447 |
| --- | --- |
| Symptom | When interworking with Polycom terminals, the RED5.0 MCU cannot send shared content to Polycom terminals due to an adaptation failure. |
| Condition | A Huawei terminal shares content in H263 XGA, but the shared content fails to be received on Polycom terminals. |
| Impact | User experience is affected. |
| Severity | Minor |
| Root Cause | The RED5.0 MCU cannot decode shared content in H263 XGA format. |
| Solution | Enable H.263-based decoding to support the SXGA format. |
| Solution Impact | None |

## Known Issues

None

# V600R019C10SPC100

## What's New

| No. | Feature | Subfeature | Description |
| --- | --- | --- | --- |
| 1 | 12-port and 24-port full switching license support on the VP9630 | - | The VP9630 supports the 12-port and 24-port full switching licenses. |
| 2 | Access of H.265 surveillance cameras in the ICP scenario | - | In the ICP scenario, H.265 surveillance cameras can be connected and H.265-based decoding is supported. |
| 3 | Serviceability optimization | - | Serviceability is optimized by: Adding a risk prompt for the reboot command Adding secondary confirmation for modifying network port configurations on the web interface Allowing users to manually clear alarms |

## Resolved Issues

None

## Known Issues

None

# V600R019C10

## What's New

New Features

| No. | Feature Name | Sub-Feature | Feature Description |
| --- | --- | --- | --- |
| 1 | Conference locking supported by SMC conference control | - | In the conference locking scenario, no endpoint (including cascaded participants) is allowed to dial into conference. The locking does not affect the other conference control functions (of the SMC or chair) including invitation. |
| 2 | DR recovery: H.323 DR supported on the SMC/SC network |  | The IP addresses of the active and standby H.323 servers can be configured. When services are switched from the active SC to the standby SC, the CloudMCU switches the H.323 registration to the standby SC. |
| 3 | On-premises EC supporting mainstream languages (10 languages) |  | The IVR announcements are in 10 languages. Chinese (Simple), Chinese (Traditional), English, French, Spanish, Portuguese, Russian, Thai, Turkish, and Italian. |
| 4 | Full switching conference supporting endpoint isolation |  | 1. Isolated participants can be defined in a conference template. After a conference is held, adaptation resources are automatically used to isolate the participants. 2. A participant can be isolated in real time in a conference by dynamically using adaptation resources. |

Modified Features

None

Deleted Features

None

## Resolved Issues

None

## Known Issues

None

# Upgrade Impact

## Impact on the System During the Upgrade

### Impact on Services

During upgrade, the system running load is high. If a conference is being held on the MCU, you are advised not to perform upgrade until the conference ends because the upgrade takes effect only after the MCU is restarted. During the restart, all services are interrupted for 5 to 6 minutes.

During the upgrade, the MCU restart time and service interruption time are as follows:

The HUAWEI VP9630 restart time is about 15 minutes.

The HUAWEI VP9650 restart time for full configuration is about 20 minutes.

The HUAWEI VP9660 restart time for full configuration is about 30 minutes.

### Impact on Network Communication

During upgrade, the communication between the MCU NE and NMS is proper. After the upgrade is successful, the MCU restarts and the service is interrupted for 15 to 30 minutes. Before the upgrade, inform the network management personnel of the upgrade time and period so that the network management personnel can ignore the alarm information generated during the period.

## Impact on the System After the Upgrade

None

# Related Documentation

## V600R019C10 Documentation

| BOM Number | Document | Description | Delivery Mode |
| --- | --- | --- | --- |
| 31500AXA | HUAWEI VP9660 V600R019C10 Installation Guide | Provides information about device installation, connection, and power supply. | Hard copy, which is delivered with the hardware device. Soft copy, which is obtained at http://enterprise.huawei.com. |
| 31500AWY | HUAWEI VP9650 V600R019C10 Installation Guide |  |  |
| 31500AWW | HUAWEI VP9630 V600R019C10 Installation Guide |  |  |
| - | VP9660 V600R019C10 Product Documentation VP9650 V600R019C10 Product Documentation VP9630 V600R019C10 Product Documentation | Provides MCU product positioning, highlights, application scenarios, and technical specifications. | Soft copy, which is downloaded at http://enterprise.huawei.com. |
| - | VP9660&VP9650&VP9630 V600R019C10 Alarm Handling | Provides device alarms and methods of handling these alarms. | Soft copy, which is downloaded at http://enterprise.huawei.com. |
| - | VP96x0&VP86x0 License Application Guide | Provides the MCU sales model, commercial license information, and license file application, loading, and upgrade operations. | Soft copy, which is downloaded at http://enterprise.huawei.com. |
| - | VP9660&VP9650&VP9630 V600R019C10 Communication Matrix | Provides the communication port of each MCU service and port description. | Soft copy, which is downloaded at http://enterprise.huawei.com. |
| - | VP9660&VP9650&VP9630 V600R019C10 Command Reference | Provides the MCU command functions, parameter meanings, and parameter value ranges. | Soft copy, which is downloaded at http://enterprise.huawei.com. |
| - | VP9660 V600R019C10 Spare Parts Document VP9650 V600R019C10 Spare Parts Document VP9630 V600R019C10 Spare Parts Document | Provides the MCU spare parts list, component codes, and component pictures. | Soft copy, which is downloaded at http://enterprise.huawei.com. |
| - | VP9660&VP9650&VP9630 V600R019C10 MIB Reference | Provides the MIB reference information that needs to be used during secondary development. | Soft copy, which is downloaded at http://enterprise.huawei.com. |
| - | VP9660&VP9650&VP9630 V600R019C10 Log Reference | Explains log reports. | Soft copy, which is downloaded at http://enterprise.huawei.com. |
| - | VP9660&VP9650&VP9630 V600R019C10 Parameter Reference | Describes the meanings of parameters that need to be configured on the Web management page, definition and description of the value range, and precautions for use. | Soft copy, which is downloaded at http://enterprise.huawei.com. |
| - | VP9660&VP9650&VP9630 V600R019C10 Acceptance Test Guide | After system installation and commissioning, an acceptance test team that consists of the customer and installation personnel carries out the acceptance test and confirms that the equipment meets requirements and specifications based on this document. | Soft copy, which is downloaded at http://enterprise.huawei.com. |
| - | VP9660&VP9650&VP9630 Release Notes | Provides information about compatible releases for the product, compatibility between software releases, updates, and resolved and remaining known issues. | Soft copy, which is downloaded at http://enterprise.huawei.com. |
| - | VP9660&VP9650&VP9630 V600R019C10 Upgrade Guide | Provides the product upgrade operations and precautions. | Soft copy, which is downloaded at http://enterprise.huawei.com. |

For details about other documents, see the solution documentation CloudVC Product Documentation.

## Obtaining Documentation

### Support-E Website

Access http://support.huawei.com/enterprise. Enter VP9660 in the search box, and view or download the required product documentation of the matching version.

You need to apply for the permission to obtain documentation from the website. If you are visiting the website for the first time, register with it. For the detailed registration procedure, see Help and FAQs on the website.

### Support Website

Log in to http://support.huawei.com/ and search for Product name + Version number, for example, VP9660 V600R006C10, to view or download your desired documents.

You need to apply for permission to obtain documentation from the website. If you are visiting the website for the first time, register with it. For the detailed registration procedure, see the help and FAQs on the website.

### Hard Copies

Find the hard copy of the Installation Guide in the accessories delivered with the product.

# Appendix

## Third-Party Software Vulnerability Fixes

None

## Acronyms and Abbreviations

| I | - |
| --- | --- |
| IP | Internet Protocol |
| M | - |
| MCU | Multipoint Control Unit |
| S | - |
| SMC | Service Management Center |