Use Net-SNMP tool to batch query the mac-address by port 0/9/0：

- Set query task

Set the index as 2, you can also set other number, to make the command:

snmpset -v 2c -c private123 10.71.51.67             //private123 is the write community, should be the same as the setting in device. And 10.71.51.67 is the device's IP.

1.3.6.1.4.1.2011.6.103.4.1.2.1.2.2 i 4                 // 2 is the index mentioned above

1.3.6.1.4.1.2011.6.103.4.1.2.1.3.2 x "00 0A 01 01 00 04 00 00 09 00"       // 00 09 00 is the hex of 0/9/0

1.3.6.1.4.1.2011.6.103.4.1.2.1.4.2 i 4               // 4 means create while 6 means destroy

So the combined command is:

snmpset -v 2c -c private123 10.71.51.67 1.3.6.1.4.1.2011.6.103.4.1.2.1.2.2 i 4 1.3.6.1.4.1.2011.6.103.4.1.2.1.3.2 x "00 0A 01 01 00 04 00 00 09 00" 1.3.6.1.4.1.2011.6.103.4.1.2.1.4.2 i 4

execute the command:

The related OIDs are:

Name:        hwBatchQueryCurrentType

Type:        OBJECT-TYPE

OID:        1.3.6.1.4.1.2011.6.103.4.1.2.1.2

Name:        hwBatchQueryParameter

Type:        OBJECT-TYPE

OID:        1.3.6.1.4.1.2011.6.103.4.1.2.1.3

Name:        hwBatchQueryRowStatus

Type:        OBJECT-TYPE

OID:        1.3.6.1.4.1.2011.6.103.4.1.2.1.4

- Get query result

Set the index as 2.0, 2 is the query task index set before, 0 means start from the first MAC ( the first time should set it as 0，if there are too many MAC，can’t get all by one time, then hwBatchQueryIsLast will return notlast(2)，need to get once more，then should set it as the sum of hwBatchQueryReturnNum ), to make the command:

snmpget -v 2c -c public123 10.71.51.67           //public123 is the write community, should be the same as the setting in device. And 10.71.51.67 is the device's IP.

1.3.6.1.4.1.2011.6.103.4.1.3.1.2.2.0

1.3.6.1.4.1.2011.6.103.4.1.3.1.3.2.0

1.3.6.1.4.1.2011.6.103.4.1.3.1.4.2.0

So the combined command is:

snmpget -v 2c -c public123 10.71.51.67 1.3.6.1.4.1.2011.6.103.4.1.3.1.2.2.0 1.3.6.1.4.1.2011.6.103.4.1.3.1.3.2.0  1.3.6.1.4.1.2011.6.103.4.1.3.1.4.2.0

execute the command within 20 seconds after execute the set query command:

the related OIDs are:

Name:        hwBatchQueryReturnNum

Type:        OBJECT-TYPE

OID:        1.3.6.1.4.1.2011.6.103.4.1.3.1.2

Description: Batch query return number

Name:        hwBatchQueryReturnList

Type:        OBJECT-TYPE

OID:        1.3.6.1.4.1.2011.6.103.4.1.3.1.3

Name:        hwBatchQueryIsLast

Type:        OBJECT-TYPE

OID:        1.3.6.1.4.1.2011.6.103.4.1.3.1.4

Description: The result of batch query is the last one or not.

Options:

1.islast(1)-last.

2.notlast(2)-not last.

In this case,

hwBatchQueryReturnNum is 3, means get 3 MACs;

hwBatchQueryIsLast is 1, means last,  no more MACs to query;

hwBatchQueryReturnList is the encoded hex message include the MACs:

SNMPv2-SMI::enterprises.2011.6.103.4.1.3.1.3.2.0 = Hex-STRING: 00 00 00 54 02 01

00 01 E4 68 A3 86 2F B0 FF FF

FF FF FF FF FF FF 00 00 09 00 FF FF FF FF FF FF

02 01 00 01 00 E0 FC 09 BC F9 FF FF FF FF FF FF

FF FF 00 00 09 00 FF FF FF FF FF FF 02 01 00 C9

E4 68 A3 86 2F B0 FF FF FF FF FF FF FF FF 00 00

09 00 FF FF FF FF FF FF

let us to decode the encoded hex message:

00 00

00 54   // total length

02   // 2 means dynamic mac while 1 means static mac

01 00 01

E4 68 A3 86 2F B0    // the first MAC

FF FF FF FF FF FF FF FF 00 00 09 00 FF FF FF FF FF FF

02   // 2 means dynamic mac while 1 means static mac

01 00 01

00 E0 FC 09 BC F9   // the second MAC

FF FF FF FF FF FF FF FF 00 00 09 00 FF FF FF FF FF FF

02  // 2 means dynamic mac while 1 means static mac

01 00 C9

E4 68 A3 86 2F B0   // the third MAC

FF FF FF FF FF FF FF FF 00 00 09 00 FF FF FF FF FF FF

And the three MAC address in the result is the same as the output in command:

The preceding command is used to query the MAC address based on port. You can also query the MAC address based on port or ONT by modifying hwBatchQueryParameter.：

  - based vlan：   1.3.6.1.4.1.2011.6.103.4.1.2.1.3.2 x "00 0A 01 03 00 04 00 64 FF FF"     // vlan 100

  - based ont:   1.3.6.1.4.1.2011.6.103.4.1.2.1.3.2 x "00 0B 01 00 00 05 00 0c 02 00 02 "    // 0/12/2 ont 2

  - based port：1.3.6.1.4.1.2011.6.103.4.1.2.1.3.2 x "00 0A 01 01 00 04 00 00 0A 02"   //0/10/2

  - base service-port：1.3.6.1.4.1.2011.6.103.4.1.2.1.3.2 x "00 0A 01 07 00 04 00 00 00 01"   // service-port  1

  - base board：1.3.6.1.4.1.2011.6.103.4.1.2.1.3.2 x "00 0A 01 02 00 04 00 00 02 FF"   // 0/2

all：1.3.6.1.4.1.2011.6.103.4.1.2.1.3.2 x "00 0A 01 04 00 04 FF FF FF FF"